Summary

This C program creates a very simple Layer-2 tunnel between a Linux TUN interface and a raw Ethernet socket using a custom EtherType (0x88B5).

It operates in two modes:

- TX mode (tx)
Reads IP packets from a TUN interface, wraps them in an Ethernet frame with a custom EtherType, and sends them over a physical Ethernet interface.

- RX mode (rx)
Receives Ethernet frames with the custom EtherType from the Ethernet interface, strips the Ethernet header, and injects the IP packet into the TUN interface.

The result is effectively a lightweight custom Ethernet encapsulation mechanism for IP traffic.

High-Level Flow

TX Path

[TUN interface]
      ↓
Read raw IP packet
      ↓
Add Ethernet header
      ↓
Send via raw socket
      ↓
[Physical NIC]

RX Path

[Physical NIC]
      ↓
Receive Ethernet frame
      ↓
Check custom EtherType
      ↓
Remove Ethernet header
      ↓
Write IP packet into TUN
      ↓
[TUN interface]

Detailed Function Commentary

Headers and Constants

#define CUSTOM_ETHERTYPE 0x88B5
#define BUF_SIZE 2000

Purpose

- CUSTOM_ETHERTYPE

  - Defines the custom Ethernet protocol identifier.

  - Frames sent by this program use EtherType 0x88B5.

- BUF_SIZE

  - Maximum packet/frame buffer size.

Notes

- 0x88B5 is in the experimental/local EtherType range.

- 2000 bytes is enough for standard MTU Ethernet frames.

tun_alloc()

static int tun_alloc(char *dev)

Purpose

Creates and configures a Linux TUN device.

What It Does

- Opens /dev/net/tun

- Requests:

  - IFF_TUN

    - TUN mode (IP packets only)

  - IFF_NO_PI

    - No extra packet metadata header

- Assigns interface name

- Returns file descriptor for packet I/O

Important Detail

TUN devices operate at Layer 3:

- Reads/writes IP packets only

- No Ethernet headers

Example

If interface is tun0:

ip tuntap add dev tun0 mode tun

Equivalent behavior is achieved programmatically here.

raw_socket_open()

static int raw_socket_open(char *iface)

Purpose

Creates a raw Ethernet socket bound to a physical interface.

What It Does

- Creates:

socket(AF_PACKET, SOCK_RAW, htons(CUSTOM_ETHERTYPE))

This allows direct Ethernet frame access.

- Retrieves interface index using:

SIOCGIFINDEX

- Binds socket to:

  - specific NIC

  - specific EtherType

Result

The socket will:

- send raw Ethernet frames

- receive only frames with EtherType 0x88B5

get_mac()

static void get_mac(int fd, char *iface, unsigned char *mac)

Purpose

Obtains MAC address of the Ethernet interface.

Mechanism

Uses:

ioctl(... SIOCGIFHWADDR ...)

Then copies:

ifr.ifr_hwaddr.sa_data

into the supplied buffer.

tx_loop()

static void tx_loop(...)

Purpose

Encapsulates TUN IP packets inside Ethernet frames.

TX Processing Steps

1. Read packet from TUN

read(tun_fd, buf, sizeof(buf))

This yields a raw IP packet.

2. Construct Ethernet header

struct ether_header *eth

Sets:

- destination MAC

- source MAC

- custom EtherType

3. Append IP payload

memcpy(frame + sizeof(struct ether_header), buf, n);

Resulting frame:

+-------------------+
| Ethernet Header   |
+-------------------+
| IP Packet         |
+-------------------+

4. Send frame

write(raw_fd, frame, txlen)

Sends directly at Layer 2.

5. Debug logging

Extracts IP header:

struct iphdr *ip = (struct iphdr *)buf;

Prints:

TX IP src -> dst len=N

rx_loop()

static void rx_loop(...)

Purpose

Receives Ethernet frames and injects IP packets into TUN.

RX Processing Steps

1. Receive Ethernet frame

read(raw_fd, frame, sizeof(frame))

2. Validate EtherType

if (ntohs(eth->ether_type) != CUSTOM_ETHERTYPE)
    continue;

Filters unrelated traffic.

3. Remove Ethernet header

unsigned char *ip_pkt =
    frame + sizeof(struct ether_header);

4. Inject packet into TUN

write(tun_fd, ip_pkt, ip_len)

The kernel now treats this as an incoming IP packet.

5. Debug logging

Prints source/destination IPs.

main()

Usage

program tx <tun> <eth> <dst-mac>
program rx <tun> <eth> dummy

TX Example

./prog tx tun0 eth0 aa:bb:cc:dd:ee:ff

- Reads packets from tun0

- Sends Ethernet frames through eth0

- Uses specified destination MAC

RX Example

./prog rx tun0 eth0 dummy

- Receives custom EtherType frames from eth0

- Injects packets into tun0

Architectural Notes

Why Use TUN?

TUN devices provide:

- virtual Layer-3 interfaces

- integration with Linux routing stack

This allows normal IP routing through the tunnel.

Why Raw Sockets?

Raw packet sockets allow:

- direct Ethernet frame access

- custom EtherType protocols

This bypasses TCP/UDP/IP transport layers.

Strengths

Simple Design

Very easy to understand:

- TUN ↔ Ethernet bridge

Efficient

Minimal encapsulation:

- only Ethernet header overhead

Good Learning Example

Demonstrates:

- TUN/TAP programming

- raw sockets

- Ethernet framing

- Linux networking internals

Weaknesses / Limitations

No Error Recovery

No:

- retransmission

- sequencing

- acknowledgements

Packet loss is unmanaged.

No MTU Handling

Large packets may:

- exceed Ethernet MTU

- fragment unexpectedly

No Validation

Assumes:

- packet contains valid IPv4

- lengths are sane

Could crash or misbehave with malformed traffic.

Unsafe Parsing

Example:

struct iphdr *ip = (struct iphdr *)buf;

No length checks before accessing headers.

Should verify:

n >= sizeof(struct iphdr)

Uses Blocking I/O

Both loops block indefinitely.

Potential improvements:

- select()

- poll()

- epoll()

No MAC Learning

Destination MAC must be manually provided.

No ARP or discovery mechanism exists.

No Security

Traffic is:

- unencrypted

- unauthenticated

Anyone on the LAN could inject fake frames.

Suggested Improvements

Add Length Validation

Example:

if (n < sizeof(struct ether_header))
    continue;

Verify IPv4 Header

Check:

- version

- IHL

- total length

Use Nonblocking I/O

Would improve responsiveness and scalability.

Add Signal Handling

Graceful cleanup on:

- SIGINT

- SIGTERM

Support Multiple Clients

Could maintain:

- MAC table

- peer list

Add Encryption

Could encapsulate:

- AES-GCM

- WireGuard-like framing

Add TAP Support

Using TAP instead of TUN would:

- carry full Ethernet frames

- preserve ARP/broadcast traffic

Overall Assessment

This is a concise and educational example of:

- Linux TUN device programming

- raw Ethernet sockets

- custom EtherType protocols

- packet encapsulation

Conceptually, it behaves like a very primitive Layer-2 VPN or Ethernet tunnel.

The code is suitable for:

- experimentation

- labs

- protocol learning

- controlled environments

But not production-ready due to:

- lack of security

- insufficient validation

- no reliability handling

- blocking architecture.

Top of Form

Bottom of Form