in the context of ATDI planning tools—HTZ Communications (for civil markets) and HTZ Warfare (for defense). My customer claims that HTZ Communications doesn't allow you to model flight tracks only HTZ Warfare does. Is this correct?

Based on ATDI's published feature descriptions, your customer's statement appears to be largely correct, with an important nuance.

What ATDI explicitly says about HTZ Warfare

ATDI repeatedly advertises flight-path and moving-platform analysis as a feature of HTZ Warfare:

- "UAV/UAS mission planning" includes flight path and elevation analysis optimisation, connectivity analysis, and counter-drone analysis based on potential flight paths.

- HTZ Warfare includes On-the-Move mission planning for air, ground, and maritime platforms.

- For UAVs, it supports:

  - waypoint-based planning,

  - path-specific coverage analysis,

  - optimisation along a flight path,

  - roll, pitch, course and antenna orientation modelling,

  - interference analysis along drone flight routes.

- ATDI also describes mission-planning functions such as "optimizing the path for a mobile seeker flying over a hostile area."

These are explicit trajectory/flight-track modelling capabilities.

What ATDI says about HTZ Communications

Public information for the civil/commercial HTZ product line focuses on:

- fixed network planning,

- coverage prediction,

- interference analysis,

- frequency planning,

- airport communications,

- air-to-ground communications,

- aeronautical propagation models,

- radar and surveillance studies.

However, in the publicly available material I could find, ATDI does not explicitly advertise flight-track or waypoint-based aircraft trajectory modelling for HTZ Communications. The flight-path, waypoint, UAV trajectory, and moving-platform features are consistently described under HTZ Warfare.

The nuance

I would be careful about saying:

"HTZ Communications cannot model flight tracks."

The public evidence only supports:

"ATDI explicitly markets and documents flight-track/waypoint trajectory analysis in HTZ Warfare, while the same capability is not clearly documented for HTZ Communications."

In practice, vendors often share a common propagation engine across civil and defence editions, with features enabled by licence. It is therefore possible that some form of trajectory analysis exists in HTZ Communications or via optional modules, but I could not find published documentation confirming that.

What I'd tell the customer

A technically defensible statement would be:

"ATDI's published documentation clearly identifies flight-path, waypoint, UAV trajectory, and on-the-move analysis as HTZ Warfare capabilities. We have not found equivalent published documentation for HTZ Communications, so if flight-track modelling is a requirement, ATDI should confirm whether it is available in the HTZ Communications licence you are considering."

That wording avoids overstating the case while aligning with the available evidence.

IF I have a call with HTZ what are the exact questions to ask to set it up for A2G network, and which parameters have to be carefully chosen?

If your objective is to determine whether HTZ Communications can fairly reproduce A2G (Air-to-Ground) predictions that you currently perform in Atoll, I'd structure the call around three areas:

- Can HTZ actually model the scenario?

- Exactly how is the model implemented?

- Which parameters most influence the results?

Below are the questions I would ask.

1. Flight Track Modelling

This is probably your first question.

Can HTZ Communications model a complete aircraft flight track, or is this capability only available in HTZ Warfare?

Follow up with:

- Can the aircraft follow an imported GPS track?

- Can altitude vary continuously along the route?

- Can the software output RSRP/SINR versus distance or time?

- Can results be exported as a CSV?

- Can predictions be generated for every point along the track?

2. Propagation Model

Since you're comparing with Atoll, this is probably the most important topic.

Ask:

Which propagation model is actually used for LTE A2G?

Specifically:

- ITU-R P.525?

- ITU-R P.526?

- Which revision?

- Is diffraction always enabled?

- Can diffraction be disabled?

- Is LOS calculated separately?

- Is Earth curvature included?

- Is atmospheric refraction included?

- What effective Earth radius (k-factor) is used?

- Can this be modified?

3. Terrain

Ask:

- Which terrain database is used?

- What terrain resolution?

- Bilinear interpolation?

- How is terrain sampled between transmitter and receiver?

- Maximum terrain profile resolution?

4. Clutter

Ask:

- Is clutter applied to airborne receivers?

- Is clutter only applied near the ground?

- At what aircraft height does clutter stop affecting predictions?

- Can clutter be disabled completely?

- Which clutter model is recommended for A2G?

5. Diffraction

This is an important one because different implementations can produce noticeably different results.

Ask:

- Which diffraction algorithm is used?

  - Deygout?

  - Bullington?

  - Delta Bullington?

  - Epstein-Peterson?

- Is this configurable?

- Is knife-edge diffraction used?

- How are multiple obstacles handled?

6. Earth Curvature

Ask:

- Is spherical Earth assumed?

- Is effective Earth radius configurable?

- Default k-factor?

- Can 4/3 Earth be changed?

7. Atmospheric Effects

Ask:

- Is standard atmospheric refraction included?

- Can refractivity be modified?

- Is evaporation ducting available?

- Is troposcatter modelled?

- Are anomalous propagation conditions available?

8. Fresnel Zone

Ask:

- Is Fresnel clearance considered?

- Which Fresnel zone?

- Does partial obstruction generate diffraction automatically?

9. Antennas

Very important.

Ask:

- Can full 3D antenna patterns be imported?

- Which formats?

- Electrical tilt?

- Mechanical tilt?

- Polarisation?

- MIMO supported?

- Cross-polar discrimination?

- Beamforming?

10. Aircraft Antenna

Ask:

- Can aircraft antenna orientation change during flight?

- Roll?

- Pitch?

- Yaw?

- Aircraft body losses?

- Dynamic antenna pointing?

11. LTE Parameters

Ask:

- How is RSRP calculated?

- Is CRS RE power used?

- Can bandwidth be specified?

- Cell load?

- MIMO gain?

- Cable losses?

- Connector losses?

- Body losses?

12. Interference

Ask:

- Does HTZ calculate

- RSRP

- RSRQ

- SINR

- CINR

- PCI

- Dominant server

- Best server

- Multi-cell interference?

13. Flight Track Outputs

Ask:

Can HTZ output

- RSRP vs distance

- RSRQ vs distance

- SINR vs distance

- Serving cell

- PCI

- Handover points

- Path profile

- LOS/NLOS status

14. Numerical Settings

This section is often overlooked but can significantly affect results.

Ask:

- Profile sampling interval?

- Maximum terrain points?

- Pixel size?

- Prediction resolution?

- Receiver sampling interval?

- Vertical interpolation?

- Horizontal interpolation?

15. Validation

Finally, I'd ask:

Has HTZ Communications been validated specifically for LTE Air-to-Ground network planning?

Follow up with:

- Against drive tests?

- Against flight trials?

- Against Atoll?

- Any published validation reports?

Parameters that most affect A2G results

If your goal is to reproduce Atoll as closely as possible, these are the parameters I'd pay the closest attention to:

| Importance | Parameter |
| --- | --- |
| ★★★★★ | Propagation model and revision (e.g. ITU-R P.525/P.526 version) |
| ★★★★★ | Diffraction algorithm and settings |
| ★★★★★ | Terrain dataset and resolution |
| ★★★★★ | Effective Earth radius (k-factor) |
| ★★★★★ | Antenna pattern (3D) |
| ★★★★★ | Electrical/mechanical downtilt |
| ★★★★★ | Aircraft altitude |
| ★★★★☆ | Clutter model and whether it applies to airborne terminals |
| ★★★★☆ | Receiver sensitivity assumptions |
| ★★★★☆ | LTE power calculation (RSRP methodology) |
| ★★★★☆ | Terrain profile sampling interval |
| ★★★☆☆ | Atmospheric refraction settings |
| ★★★☆☆ | Fresnel zone treatment |
| ★★★☆☆ | Prediction grid resolution |

If your objective is a direct comparison with Atoll

I'd end the meeting with one key question:

"Can you provide the complete list of propagation model settings and numerical parameters used by HTZ Communications for A2G predictions so that we can reproduce the calculations as closely as possible in Atoll (or vice versa)? This includes the exact ITU-R recommendations and revisions, diffraction method, Earth curvature assumptions, terrain profile sampling, clutter treatment, and any numerical implementation details."

That question is likely to yield the most useful information for explaining or reconciling differences between HTZ and Atoll predictions.