Propagation Models ITU-R P525/ P526 in HTZ and Atoll (WP198)

- Assessment ...............

NOTUS Mobility Testing at Regent’s Park (WP209)

- Assessment ...............

Cluster Test – Northeast To Southwest Cluster Coverage Analysis (WP223)

- Assessment ...........

Cluster Test – Scottish Islands Transit Route

- Active Measurements (WP244)

- Passive Measurements (WP248)

TTO Reports (Transition To Operations)

- Passive testing through two orbits: Main and Standby eNodeBs

- Assessment of power levels, signal quality, presence of expected PCs, beam squints, intra-handovers.

- Analysis of raw data, filtered data, and predicted power levels.

Tested sites:

- Pen Y Forest TTO (WP222)

- Belmont TTO (WP224)

- Waltham TTO (WP225)

- BT Tinshill TTO (WP226)

- Long Mountain TTO (WP227)

- WERFA TTO (WP229)

- Gaer Hill TTO (WP230)

- Cleeve Hill TTO (WP231)

- Pen Hill TTO (WP232)

****************************************************************

COL Tests (Critical Operational Locations)

- Assessment of reach, coverage and resilience of the ESN Air network in a demanding environment.

- Active testing for all bands: Band3, Band 20, Band 40.

- Two Reports:

  - Short end-user report: KPI parameters, MOS and CSR over tested area. iPerf more recently

  - Long in-depth report

    - Heatmaps of results across height

    - PCI distribution over the area flown and across height

- Tested locations:

  - HELIOS Phase 7

    - Caernarfon (WP200/WP201)

  - HELIOS Phase 8

    - Eaglescott (WP217/WP219)

    - Henstridge (WP218/WP221)

  - HELIOS Phase 9:

    - Exeter Airport (WP211/WP212)

    - Newquay (WP213/WP214)

    - Nottingham (WP215/WP216)

COL Tests (Critical Operational Locations)

- Tested locations:

  - HELIOS Phase 10

  - North Weald (WP245)

  - Earls Colne (WP2046)

  - HELIOS Phase 11 -- WIP

  - Swansea (WIP)

  - Cardiff (WIP)

COL Tests (Critical Operational Locations)

- No predictions

- Rectified issues COL tests - TESTING (ag)

- 12dB loss primarily affects RSRP - But can impact handovers

- TTO quite consistent across except for isolated cases

- COL sites all work, network layers are complementary

- Mobility???? → always external issue that stops us from analysing mobility with confidence (incomplete deployments (B3 primarily), 12dB loss, ...)

- Incomplete deployments (or partially deployed) remove value from analysis and reports

- Justifying why it wasn't as good as it was expected

- Clusters - consistency with previous clusters in cell-site selection strategy

****************************************************************

HTZ

For a 1-minute presentation, very high level and avoid mentioning all the ITU recommendation numbers unless someone asks. Here's a concise version:

Aim / Objectives

- Compare Atoll and HTZ propagation models.

- Understand differences between old and new ITU models.

- Assess the impact on network prediction accuracy.

Main Conclusions

- Free-space calculations are effectively the same.

- Differences appear when terrain and obstacles are included.

- Newer HTZ models provide more advanced diffraction modelling.

Lessons Learned

- Model choice affects prediction accuracy.

- Older models remain usable but have limitations.

- Updated models are better suited to complex environments.

Aim / Objectives

"This report compares the propagation models used in Atoll and HTZ. The objective was to understand the differences between the older and newer ITU models, and to assess whether those differences could affect network prediction accuracy."

Main Conclusions

"We found that the free-space calculations are essentially the same between the models. The main differences appear when terrain and obstacles are considered. The newer HTZ models include more advanced diffraction methods, which can provide more realistic predictions in complex environments."

Lessons Learned

"The main lesson is that the choice of propagation model can influence prediction accuracy. While the older models are still suitable for many scenarios, the newer models offer advantages where terrain is more challenging."

****************************************************************

Scottish

The slides should answer three questions:

- Why did we do the work?

- What did we assess?

- What did we learn?

Here is a concise version.

Report

- Airborne LTE performance assessment over the Scottish Islands

- Comparison of active UE and passive scanner measurements

- Evaluation of terrestrial (Bands 3/20) and A2G (Band 40) network performance

- Assessment conducted at 1,500 ft under representative operational conditions

Aim / Objectives

- Assess RF coverage and signal quality

- Evaluate VoLTE service continuity and accessibility

- Compare active and passive measurement results

- Assess the contribution of Bands 3, 20 and 40 to airborne coverage

- Identify coverage gaps and opportunities for network optimisation

Key Lessons Learnt

- Band 3 provided the most consistent operational performance during the trial.

- Band 20 delivered stronger RSRP but did not translate into significantly better operational performance.

- Band 40 provided excellent local RF quality but coverage continuity was limited by A2G coverage gaps.

- Passive measurements confirmed that the artificial test loss primarily affected RSRP, with much smaller impact on RSRQ and SINR.

- Active measurements alone can bias comparisons between Bands 3 and 20 due to network band-selection behaviour.

- Completion of the remaining A2G sites is expected to improve airborne coverage continuity.

- Higher-altitude testing is recommended to validate Band 40 standalone performance as reliance on the terrestrial layer reduces.

These bullets are suitable for an executive/customer audience: they convey the key messages without delving into detailed RF metrics or assumptions that belong in the report.

Report

"This report presents the results of the airborne LTE trials carried out over the Scottish Islands. We compared active UE measurements with passive scanner measurements to assess the performance of the terrestrial network and the Air-to-Ground network."

Aim / Objectives

"The main objectives were to assess RF coverage and signal quality, evaluate VoLTE performance and service continuity, compare active and passive measurements, and identify any coverage gaps or areas for optimisation."

Key Lessons Learnt

"The main findings were that Band 3 provided the most consistent operational performance during these trials, while Band 20 generally provided stronger signal strength but not significantly better service. Band 40 delivered very good RF quality close to A2G sites, although some coverage gaps remain. The passive measurements also confirmed that the artificial test loss mainly affected signal strength rather than signal quality, and highlighted the importance of using both active and passive measurements to fully understand network performance."

45–60 seconds

*********************************************************************************

WIDE BEAM antenna (MIXED??? ChatGPT)

Aim / Objectives

- Assess airborne performance of the wide-beam Band 40 antenna.

- Evaluate RF coverage, VoLTE service continuity and mobility.

- Compare active UE and passive scanner measurements.

- Assess the contribution of the terrestrial (Bands 3/20) and A2G (Band 40) layers.

- Identify coverage gaps and opportunities for network optimisation.

Script

"The aim of the trials was to assess the performance of the wide-beam antenna under operational conditions. We evaluated RF coverage, service continuity and mobility, compared active and passive measurements, and assessed how the terrestrial and A2G layers work together."

Conclusions

- Band 3 provided the most consistent operational performance at 1,500 ft.

- Band 40 delivered excellent RF quality close to A2G sites but exhibited coverage gaps between sites.

- Neighbouring A2G sites did not provide sufficient backfill to eliminate the coverage gaps of the target wide-beam sector.

- Short-call testing showed higher call setup failures, indicating accessibility limitations.

- The most challenging areas were the long sea crossings across The Minch and the Hebrides.

Script

"The main conclusion is that Band 3 provided the most consistent service during these trials. Band 40 performed very well close to A2G sites, but coverage gaps remained and neighbouring A2G sites did not fully backfill those gaps. We also found that short calls experienced more setup failures, suggesting accessibility is a key area for improvement."

Lessons Learnt

- Passive measurements helped explain the active UE results.

- Artificial test loss mainly affected RSRP, with much smaller impact on RSRQ and SINR.

- Active measurements alone can bias comparisons between Bands 3 and 20 due to network band-selection behaviour.

- Ground-layer performance is expected to reduce with increasing altitude.

- Future airborne coverage will depend increasingly on the Band 40 A2G network, making completion of the A2G deployment important.

Script

"The passive measurements gave us a better understanding of the active results. We confirmed that the artificial loss mainly reduced signal strength, while signal quality was affected much less. As aircraft fly higher, the terrestrial layer is expected to become less effective, so future airborne coverage will increasingly rely on the Band 40 A2G network."

I would avoid discussing the detailed 12 dB loss, RSRQ wideband vs narrowband, or handover algorithm bias in a customer presentation unless specifically asked. Those are excellent technical findings for the report, but for a presentation the audience is more likely to remember the higher-level messages: what was assessed, what was found, and what it means for the A2G network.

****************************************************************************

Apart from wide-beam antenna and scottish islands which other reports or analyses were discussed?

The main ones are:

1. Wide-Beam Antenna Evaluation (Active Tests)

- Comparison of wide-beam Band 40 performance with Bands 3 and 20.

- RF assessment (RSRP, RSRQ, SINR).

- Operational assessment (MOS, accessibility, retainability, mobility).

- Coverage continuity and neighbouring A2G site backfill.

- Executive summary, conclusions and recommendations.

- Presentation slides and speaker notes.

2. Wide-Beam Antenna Evaluation (Passive Scanner Tests)

- Comparison of passive and active measurements.

- Effect of the artificial 12 dB test loss.

- Differences between RSRP and RSRQ/SINR behaviour.

- Band 3 versus Band 20 selection bias.

- Band 40 coverage potential.

- Impact of BIS (Brought Into Service) and partially operational A2G sites.

- Recommendations regarding band selection and future testing.

3. Scottish Islands Flight Campaign (500 m / 1,500 ft)

- Prestwick ↔ Inverness routes.

- Long VoLTE call versus short intermittent calls.

- Comparison of Band 3, Band 20 and Band 40.

- Accessibility versus retainability.

- Uplink versus downlink performance.

- Operational hotspots (The Minch, Hebrides, Islay/Jura).

- Executive summary, conclusions, recommendations and presentation material.

4. Drone Coverage Validation – Earls Colne

- Vertical ascent/descent with circular orbits.

- Altitude-dependent performance of Bands 3, 20 and 40.

- RF and operational assessment.

- Review and correction of a colleague's report.

- Discussion of complementary terrestrial and A2G coverage.

5. Drone Coverage Validation – North Weald

- Similar drone trial methodology.

- Altitude transition between terrestrial and A2G layers.

- Review and improvement of report wording.

- Assessment of the combined network concept.

6. HTZ Communications vs HTZ Warfare Analysis

- Whether HTZ Communications supports airborne flight tracks.

- Questions to ask ATDI regarding A2G planning.

- Appropriate propagation models.

- Parameters required for A2G network modelling.

7. Terrestrial Propagation Model Comparison

- Comparison between HTZ and Forsk Atoll.

- ITU-R P.525/P.526 implementation differences.

- Assessment of propagation model suitability.

- Preparation of presentation script.

8. Passive vs Active PCI / Band Analysis

- Discussion of missing PCI plots.

- Importance of PCI plots for assessing neighbour backfill.

- Interpretation of serving-band plots versus PCI plots.

- Recommendations for improving the analysis.

From the amount of work we've done, I'd say the four major technical reports are:

- Wide-Beam Antenna Trial (Active).

- Wide-Beam Antenna Trial (Passive).

- Scottish Islands Airborne Flight Trial.

- Drone Validation Trials (Earls Colne & North Weald).

The other topics (HTZ, propagation models, PCI analysis) were more focused technical investigations or supporting analyses rather than standalone reports.