Routing_table_netlink_impl

Yes. The design makes sense, and importantly, the additional code you've shown confirms that the Linux port is doing more than merely reading the routing table: it is also configuring neighbour/L2 information through Netlink.

I also don't agree with the blanket statement that it can only be verified on the fully deployed target. A substantial part can be validated on a Linux VM before system-level testing, although a VM cannot prove target-specific integration.

What this code is doing

There are now two related Netlink operations:

Higher-level configuration

|

v

routing_table_netlink_impl

/           \

/             \

routing table       neighbour table

RTM_*               RTM_*

|                   |

v                   v

IP destination       L2/MAC address

gateway/interface   of neighbour

The important new functions are:

prepare_ndmsg_header()

make_rtm_neighbour_message()

send_rtm_neighbour_message()

Those are clearly intended to construct and send neighbour-table Netlink messages using struct ndmsg.

So the comment:

"Recently Modified to set the Layer 2 neighbour addresses..."

is consistent with the implementation.

The routing part uses:

struct rtmsg

while the neighbour part uses:

struct ndmsg

That's the correct general Linux Netlink architecture.

Can this be tested in a VM?

Yes. Definitely at several levels.

You don't need the final target hardware just to determine whether these Netlink messages are correctly constructed and whether Linux accepts them.

For example, a normal Linux VM can have:

VM

|

+-- eth0

|     IP: 192.168.1.100

|

+-- Linux routing table

|

+-- Linux neighbour table

|

+-- NETLINK_ROUTE

You can then run the application and verify that:

1. Routes are actually installed

Before:

ip route

Run your:

RT_update_routing_table(...)

Then:

ip route

and verify that the expected routes appear.

2. Neighbour/L2 entries are actually installed

Before:

ip neigh

Run the code.

Then:

ip neigh

and verify the expected neighbour entry.

For example, something conceptually like:

192.168.1.1 dev eth0 lladdr xx:xx:xx:xx:xx:xx ...

That would directly test whether your Netlink neighbour configuration is working.

3. Error handling can be tested

Your:

send_routing_msg()

and:

send_rtm_neighbour_message()

can be tested with deliberately invalid routes/neighbours.

You can verify that the kernel returns the expected Netlink errors and that your code handles NLMSG_ERROR correctly.

What cannot be completely validated in a VM

This is where your colleague has a valid point, but it is much narrower than "you can't verify it".

A VM cannot prove that the complete deployed system will work because you still need to test things such as:

- the target's actual network interfaces

- the target's actual interface names/indexes

- the target routing configuration

- the actual protocol-driver interface

- interaction with your target hardware

- the actual L2 network

- timing/concurrency with the rest of the system

- target-specific kernel/network configuration

- the interaction between this management software and your ported driver

So I'd distinguish:

| Test | VM? |
| --- | --- |
| Construct Netlink route message correctly | Yes |
| Kernel accepts route | Yes |
| Route appears in ip route | Yes |
| Construct neighbour message correctly | Yes |
| Kernel accepts neighbour entry | Yes |
| Entry appears in ip neigh | Yes |
| Route lookup works | Yes |
| Netlink error handling | Yes |
| Management application's routing logic | Yes |
| Interaction with your Linux protocol driver | Yes, with suitable mock/TAP setup |
| Actual target interfaces/hardware | No |
| Complete deployed-system behaviour | No |

One particularly useful test

You could create a self-contained Linux VM integration test with a virtual Ethernet topology:

VM

+--------+

|        |

| eth0   |

|        |

+---+----+

|

veth pair

|

+---+----+

|        |

| Linux  |

| peer   |

+--------+

Then your management application can:

- Add a route using your RT_update_routing_table().

- Add the neighbour/L2 address using your Netlink code.

- Query the resulting routing/neighbour state.

- Send traffic.

- Verify that the expected interface and next hop are selected.

That gives you a real Linux kernel, real Netlink, real routing table, and real neighbour table — without needing the final target.

And this is particularly relevant to your TUN/TAP question

You can go one step further with TAP:

Management application

|

Netlink

|

v

Linux routing +

neighbour tables

|

v

TAP

|

v

Protocol-driver test harness

That would allow you to validate both:

routing/neighbour configuration and L2 packet handling in the VM.

So I would not wait until target deployment to test this code. I'd use the VM to establish that the Netlink implementation works against a real Linux kernel, then use system-level testing to validate the target-specific integration.

One caveat: your snippet says some functions require CAP_NET_ADMIN. That's also easy to test in a VM; you can run the executable with the capability or run the test with the appropriate privileges.

what is the function to Add the neighbour/L2 address using my Netlink code

From the code you provided, the function that ultimately sends the neighbour/L2 address configuration to Linux is:

send_rtm_neighbour_message(file_des_t routing_socket)

However, the function that constructs/prepares the neighbour/L2 Netlink message is:

make_rtm_neighbour_message(

const int_4 cmd,

int if_index,

ipv4_addr_t dest_address)

The likely sequence is:

route_action()

|

+--> make_rtm_neighbour_message()

|       |

|       +--> prepares struct ndmsg + attributes

|

+--> send_rtm_neighbour_message()

|

+--> sends Netlink message to Linux

There is also:

prepare_ndmsg_header(struct ndmsg *ndm, int if_index)

which prepares the struct ndmsg header.

But there is an important point

From the prototypes you've shown, I haven't yet seen where the actual MAC/L2 address is supplied.

make_rtm_neighbour_message() takes:

cmd

if_index

dest_address

but no L2/MAC address parameter.

Therefore I would want to see the implementation of:

make_rtm_neighbour_message()

because it presumably adds the L2 address as a Netlink attribute, probably something equivalent to:

NDA_LLADDR

possibly obtaining the address from the rte_t or some other global/state.

So, if your question is "which function in this code actually adds the L2 neighbour address?", the answer is most likely:

make_rtm_neighbour_message() constructs it, and send_rtm_neighbour_message() sends it to the Linux kernel.

If you paste the implementation of make_rtm_neighbour_message(), I can tell you exactly where the L2 address comes from and whether the implementation is actually sufficient.