I think when I initially tested this it worked (I may be wrong) but now this is the output I get at the end of the test. What could have gone wrong? My output is:

========== START TEST ==========

Linux configuration complete.

Waiting for an Ethernet frame on tap0...

==============================
PROTO READ FROM TAP
===============================

Frame length: 70 bytes

Ethernet header:

- Destination MAC: 33:33:00:00:00:02

- Source MAC: 02:00:00:00:00:01

- COMSYS L2 address: 2

IP header:

- Destination IP: 0.0.0.255

Expected result:

- Final L3 destination: 10.0.1.99

- L2 address: 2

**********************************************************************************

Yes — the output actually gives a very strong clue, and I think the test itself is probably still working. The problem is that proto is reading the first Ethernet frame that Linux sends on tap0, and that frame is not your test IPv4/UDP packet.

The key evidence is:

Destination MAC: 33:33:00:00:00:02

Frame length: 70 bytes

Destination IP: 0.0.0.255

What is happening

33:33:00:00:00:02 is an IPv6 multicast Ethernet address. In particular:

33:33:00:00:00:02

corresponds to the IPv6 multicast address:

ff02::2

which is the IPv6 all-routers multicast address.

A 70-byte Ethernet frame is also very plausible for an IPv6 Router Solicitation containing a source-link-layer option:

Ethernet       14 bytes

IPv6 header    40 bytes

ICMPv6         16 bytes

----

70 bytes

So what has almost certainly happened is:

- You created tap0.

- You brought tap0 up.

- Linux's IPv6 stack generated an IPv6 control packet.

- That packet appeared on TAP.

- Your C program did:

read(tap_fd, buffer, sizeof(buffer));

- It stopped at that first frame.

- Your C code then interpreted the bytes following the Ethernet header as an IPv4 header.

- Consequently it printed:

Destination IP: 0.0.0.255

which isn't the real destination IP of the frame.

So the important point is:

The TAP mechanism has not failed. Your test is reading an IPv6 control frame before it gets to the IPv4 test packet.

Why the MAC address is the giveaway

Your intended packet should be:

Destination MAC: 02:00:00:00:00:02

Source MAC:      02:00:00:00:00:01

Instead you got:

Destination MAC: 33:33:00:00:00:02

Source MAC:      02:00:00:00:00:01

The source MAC is exactly what we expect from tap0.

The destination MAC is completely different because this is an IPv6 multicast packet.

Also notice that your C program currently apparently doesn't print the Ethernet EtherType. I would add that immediately, because it would make this problem obvious:

EtherType: 0x86DD

0x86DD = IPv6.

Your intended packet should have:

EtherType: 0x0800

0x0800 = IPv4.

The simplest fix

I would not change the Linux routing setup.

Instead, make proto ignore frames that aren't IPv4 and continue reading until it gets the frame you actually want to test.

Conceptually:

for (;;) {

len = read(tap_fd, buffer, sizeof(buffer));

struct ethhdr *eth = (struct ethhdr *)buffer;

printf("EtherType: 0x%04x\n", ntohs(eth->h_proto));

if (ntohs(eth->h_proto) != ETH_P_IP) {

printf("Ignoring non-IPv4 Ethernet frame\n");

continue;

}

/* Now parse IPv4 header */

break;

}

Then your output should eventually become:

EtherType: 0x86dd

Ignoring non-IPv4 Ethernet frame

EtherType: 0x0800

PROTO READ FROM TAP

Frame length: ...

Ethernet header:

Destination MAC: 02:00:00:00:00:02

Source MAC:      02:00:00:00:00:01

COMSYS L2 address: 2

IP header:

Destination IP: 10.0.1.99

That is actually a better demonstration of TAP because it shows that TAP provides the complete Ethernet frame, not just the particular IP packet you're interested in.

Alternatively, disable IPv6 for this test

If you want the demo to be absolutely clean and only produce your intended IPv4 traffic, add this to the bash configuration:

sysctl -w net.ipv6.conf.tap0.disable_ipv6=1

Then configure the rest:

ip link set dev tap0 address 02:00:00:00:00:01

ip link set dev tap0 arp off

ip addr add 192.168.0.1/24 dev tap0

ip link set dev tap0 up

ip neigh replace 192.168.0.2 \

lladdr 02:00:00:00:00:02 \

nud permanent dev tap0

ip route replace 10.0.1.99/32 \

via 192.168.0.2 \

dev tap0

However, I would still recommend checking the EtherType in the C code. In a real TAP-based production implementation, you cannot assume that every frame arriving at TAP is IPv4.

There is another useful test

Before changing anything, run:

ip -d link show tap0

and:

ip -6 addr show dev tap0

You'll probably find that IPv6 is enabled on tap0.

You can also see what Linux is sending with:

tcpdump -e -n -i tap0

You will likely see something resembling:

33:33:00:00:00:02 > 02:00:00:00:00:01, ethertype IPv6 ...

followed later by your intended:

02:00:00:00:00:02 > 02:00:00:00:00:01, ethertype IPv4 ...

The latter is the frame we're interested in.

Most importantly: your original L2/L3 demonstration is still valid

Once the IPv6 frame is skipped, the important relationship should be:

Linux routing

|

v

Final IP       10.0.1.99

|

route via

|

v

Next-hop IP    192.168.0.2

|

neighbour lookup

|

v

Next-hop MAC   02:00:00:00:00:02

|

v

Ethernet frame

|

v

TAP

|

v

proto

Therefore proto gets:

Ethernet destination MAC = 02:00:00:00:00:02

COMSYS L2 address         = 2

IP destination            = 10.0.1.99

That is exactly the distinction you wanted to demonstrate: the Ethernet destination/L2 identity belongs to the next hop, while the IP destination remains the final destination.

I would therefore make one small improvement to the C demo rather than changing the topology: have it print the EtherType and skip non-IPv4 frames. That makes the test robust and also makes it much clearer what TAP is actually delivering.