Yes. There are several important points in these results, and I think the data actually gives you a fairly strong indication of where the problem is.

The most important distinction is this:

H and V RSRP do not need to be equal at a fixed location. However, if both FVDs are connected to the same antennas and the RF paths are equivalent, you would expect the two FVDs to give similar results for the same cell. The old configuration demonstrates that this is achievable.

The fact that the old configuration gives almost identical FVD1/FVD2 RSRP, while the new configuration does not, is particularly significant.

1. PENTSATENDOTENA  the results are telling you

Nik would separate the observations into three effects:

- The B40 H/V imbalance is real and can legitimately be large.

- The FVD1/FVD2 imbalance introduced by the new configuration is not explained simply by the H/V characteristics of the antenna.

- The new configuration appears to have introduced unequal RF paths, and the cable-swap result strengthens that suspicion.

The old configuration is effectively a very useful reference because both FVDs receive the signal through the same combiner/splitter chain.

For example:

| Band | Old FVD1 | Old FVD2 | Difference |
| --- | --- | --- | --- |
| B3 | -98.66 | -98.79 | 0.13 dB |
| B20 | -73.44 | -73.19 | 0.25 dB |
| B40 | -86.92 | -87.04 | 0.12 dB |

Those are remarkably close.

By contrast, with the new configuration:

| Band | New FVD1 | New FVD2 | Difference |
| --- | --- | --- | --- |
| B3, Test 6 | -85.06 | -79.44 | 5.62 dB |
| B20, Test 7 | -63.69 | -68.81 | 5.12 dB |
| B40, Test 8 | -82.06 | -84.51 | 2.45 dB |

That is not a small measurement difference.

And after the cable swap:

| Band | New FVD1 | New FVD2 | Difference |
| --- | --- | --- | --- |
| B3, Test 13 | -84.54 | -82.24 | 2.30 dB |
| B20, Test 14 | -69.75 | -65.10 | 4.65 dB |
| B40, Test 15 | -78.78 | -84.81 | 6.03 dB |

The important observation is that the imbalance changes substantially when the cables are swapped.

That is exactly the sort of behaviour Nik would investigate as a path-dependent RF issue, rather than immediately attributing it to propagation.

2. Your point about V and H not needing to be balanced is correct

Yes — I agree with your understanding here.

At a particular static location, there is no reason to expect:

RSRP(V) = RSRP(H)

The two polarizations can experience different:

- antenna gain;

- antenna radiation pattern;

- polarization alignment;

- cross-polar discrimination;

- elevation/azimuth response;

- multipath;

- reflections;

- diffraction;

- propagation geometry;

- eNodeB antenna characteristics.

And these effects can change significantly as the aircraft/vehicle moves.

So I would not describe an H/V difference as a problem simply because one polarization is 10 dB or 15 dB stronger than the other.

In fact, your old Band 40 results demonstrate this very nicely: the H/V difference is around 17 dB, while B3/B20 are much closer.

That can quite legitimately be an antenna/propagation characteristic.

But there is an important second point

The fact that H and V do not have to be equal does not mean that the two FVDs should have different results.

If FVD1 and FVD2 are both receiving the same physical antenna signals, then under otherwise identical conditions you would expect:

FVD1 total received signal ≈ FVD2 total received signal

even though:

V ≠ H

So I would correct your colleagues' interpretation in exactly that way.

3.  would be careful with the phrase "combined V+H should be the same"

Your principle is right, but I would slightly change the terminology.

I would not necessarily say:

"V + H RSRP should be the same."

RSRP is a logarithmic quantity and the UE's reported RSRP/antenna measurements are not necessarily intended to be arithmetically added in dBm.

Also, if XCAL is showing Ant0 and Ant1, those are measurements on the individual receiver branches. The UE may use receiver combining internally for demodulation, but that does not mean that:

RSRP(Ant0) + RSRP(Ant1)

is a meaningful "combined RSRP".

What you really want to establish is:

For the same serving cell, at the same location and with the same antenna configuration, do FVD1 and FVD2 have equivalent end-to-end RF performance?

The old configuration gives you a baseline showing that they can be within ~0.1–0.3 dB.

That is much more compelling than requiring V and H themselves to be equal.

4. The really interesting result is actually Table 3

I think Table 3 deserves more attention than the individual FVD RSRP numbers.

For the old configuration, Band 40 has approximately:

- FVD1: 16.6–17.4 dB

- FVD2: 16.9–17.9 dB

So the B40 antenna branches have a very large and very consistent H/V imbalance.

That by itself is not necessarily a problem.

But in the new configuration you have:

- FVD1: -9.54 dB

- FVD2: -7.20 dB

- later: -10.42 dB

- later: -6.5 dB

So the apparent B40 H/V relationship has effectively reversed.

That is much more significant than simply saying "Band 40 has a large V/H imbalance."

If the same physical Band 40 antenna and same eNodeB are being used, then the change from approximately:

V − H ≈ +17 dB

to approximately:

V − H ≈ −7 to −10 dB

cannot sensibly be explained by "Band 40 is an Air-to-Ground antenna."

The antenna/propagation characteristics can explain why B40 may have a larger H/V imbalance than B3/B20.

They cannot, by themselves, explain why the imbalance changes from approximately +17 dB to approximately -8 dB when you change the RF distribution network.

That points much more strongly toward something in the RF routing/configuration or measurement chain.

5. The cable-swap experiment is particularly important

I think Tests 13–15 are one of the strongest pieces of evidence you have.

Before the swap:

- B3: FVD1 was 5.62 dB lower than FVD2.

- B20: FVD1 was 5.12 dB higher than FVD2.

- B40: FVD1 was 2.45 dB higher than FVD2.

After the swap:

- B3: FVD1 is 2.30 dB lower than FVD2.

- B20: FVD1 is 4.65 dB higher than FVD2.

- B40: FVD1 is 6.03 dB higher than FVD2.

So the discrepancy is not fixed to a particular FVD.

That is important.

It suggests that the cause may be associated with the RF path being swapped, or with an interaction between the path and the FVD input.

However, I would not yet conclude "the cable is faulty". You have several things travelling together:

- cable;

- connector;

- diplexer;

- diplexer pair;

- FVD RF input;

- potentially antenna feed.

A cable swap alone doesn't isolate which component is responsible.

6. This is exactly what I would expect the old/new architectures to expose

There is a fundamental architectural difference between your two configurations.

Old configuration

You effectively have:

Antenna → combiner → splitter → FVD1

and

Antenna → combiner → splitter → FVD2

The splitter creates two nominally identical outputs.

Therefore, any insertion loss in the common path is experienced by both FVDs.

If the common path has, say, 6 dB of loss, both FVDs see approximately 6 dB of loss.

That is why you get:

FVD1 ≈ FVD2

even if the absolute RSRP is poor.

New configuration

You instead have:

Antenna → diplexer pair 1 → FVD1

and

Antenna → diplexer pair 2 → FVD2

Now there is no longer one common RF path.

Any difference between:

- diplexer pair 1 and pair 2;

- connectors;

- cables;

- insertion loss;

- return loss;

- isolation;

- frequency response;

will directly appear as a difference between FVD1 and FVD2.

This is, in my view, a very plausible explanation for the change you are seeing.

7. But the size of the new-vs-old gains is suspicious

This is another thing I would highlight.

For the directly comparable B3 tests:

FVD1

- Old: -98.66

- New: -85.06

- apparent gain: +13.60 dB

FVD2

- Old: -98.79

- New: -79.44

- apparent gain: +19.35 dB

And B20:

- FVD1: +9.75 dB

- FVD2: +4.38 dB

The average apparent gain is therefore approximately:

- B3: 16.5 dB

- B20: 7.1 dB

If the only intended change is removal of combiner/splitter loss and replacement by diplexers, I would want to compare those numbers against the actual measured insertion loss of both architectures.

If your laboratory measurements say that the old path is, for example, only 5–7 dB worse than the new path, then a measured field improvement of 10–19 dB cannot be attributed to the passive hardware alone.

That would be a strong indication that the sequential field measurements are being affected by something else.

8. I would not blame "live traffic" for the B3/B20 idle-test discrepancies

This is an important point.

Tests 1–3 and 6–8 are all:

FVD1 = Idle
FVD2 = Idle

So the 5–6 dB FVD-to-FVD differences in Tests 6–8 cannot be explained by the continuous-call conditions.

Also, LTE RSRP is based on reference signals rather than the total traffic power, so ordinary traffic loading should not normally produce a 5–10 dB change in RSRP simply because the cell is busier.

Traffic can affect other measurements such as:

- RSRQ;

- SINR;

- throughput;

- scheduling;

- receiver behaviour in some circumstances.

And, importantly in your setup, UE transmission can cause RF desensitisation/interference if the RF isolation is inadequate.

That is especially relevant to your B40 TDD case.

But it does not explain the fundamental FVD1/FVD2 difference in Tests 6–8 because those tests are idle.

So I would treat the idle results as the cleanest evidence that there is an underlying issue with the new RF paths.

9. Test 11 should not be used as strong evidence for the expected gain

I agree with your caution here.

Test 11 has:

- FVD1 on PCI 30;

- FVD2 initially on PCI 31;

- FVD2 subsequently on PCI 30;

- both FVDs in continuous call.

The second FVD2 measurement on PCI 30 is useful because it gives you a same-cell comparison, but it is still not equivalent to Test 3 because the operating condition is different.

You effectively have:

Test 3

Old configuration + PCI 31 + both FVDs idle

versus

Test 11

New configuration + PCI 30/31 + active calls

and then FVD2 happens to measure PCI 30 later.

So the fact that the PCI-31 measurement is around -87 dBm in both cases is interesting, but I would not use it to conclude that the new configuration provides no gain.

There are too many changed variables.

10. Could the Band 40 antenna explain the large H/V delta?

Yes, partly.

I think your hypothesis is reasonable.

A dedicated A2G antenna can have a very different radiation pattern from a terrestrial cellular antenna. The relative response of the two polarizations can therefore be very different at a particular ground location.

Also, Band 40 is at approximately 2.3 GHz, while:

- B3 is around 1.8 GHz;

- B20 is around 800 MHz.

The antennas, propagation and polarization behaviour are therefore not directly comparable.

At a fixed location, you could quite legitimately have:

B3: H ≈ V

B20: H somewhat different from V

B40: H very different from V

So I would not try to force the B40 H/V difference to match B3/B20.

But again, the crucial observation is that the old configuration gives a very repeatable ~17 dB B40 difference for both FVDs.

That tells you that the large B40 H/V difference can be a genuine characteristic of the RF environment/antenna.

The fact that the new configuration changes that relationship so dramatically is a separate issue.

11. Test 16 actually makes sense — but not in the way your colleagues seem to be interpreting it

Your rotation test is interesting.

You observed approximately:

Initial orientation

FVD1 = -87
FVD2 = -87

60°

FVD1 = -80
FVD2 = -87

215°

First part:

FVD1 = -80
FVD2 = -85

Second part:

FVD1 = -80
FVD2 = -80

There are two different effects here.

Effect 1 — antenna rotation changing RSRP

That absolutely makes sense.

Changing the orientation of a directional/polarised antenna can change received power significantly.

So going from:

-87 → -80 dBm

is not surprising.

Effect 2 — FVD1 and FVD2 responding differently

This is the interesting part.

If both FVDs are connected to the same physical antenna signal through nominally equivalent RF paths, rotating the antenna should cause both FVDs to follow the same general trend.

They don't necessarily have to produce exactly the same number, but a repeatable 7 dB change in one while the other doesn't change is something worth investigating.

12. The repeated 215° result is particularly useful

This part is actually quite telling.

At 215° you apparently obtained:

First observation

- FVD1 = -80

- FVD2 = -85

but later:

Second observation

- FVD1 = -80

- FVD2 = -80

The antenna had not changed orientation between those two measurements.

Therefore, the 5 dB difference in FVD2 cannot be explained simply by the antenna orientation.

Something else changed.

Possibilities include:

- RF path/connector movement;

- FVD receiver state;

- XCAL measurement state;

- serving-cell measurement/filtering;

- network state;

- UE RF calibration/AGC state;

- environmental multipath changes;

- another RF transmission/interference source.

But because the test is static, I would not use the rotation results as evidence that the antenna itself is responsible for the FVD discrepancy.

In fact, the repeatability issue makes them useful evidence that you need a more controlled experiment.

13. One experiment would tell you much more than more field tests

I would recommend doing a conducted/common-source test before spending much more time trying to interpret the field results.

The key question is:

Do diplexer path 1 and diplexer path 2 actually deliver the same RF power to FVD1 and FVD2?

You can answer that independently of:

- eNodeB;

- propagation;

- antenna orientation;

- traffic;

- PCI;

- weather;

- multipath.

For each band, ideally measure:

common RF source → diplexer path → FVD input

for:

- Path 1;

- Path 2.

You could use a calibrated RF source/spectrum analyser/power meter arrangement appropriate to your equipment and safe input levels.

Do it independently for:

- B3;

- B20;

- B40;

and for:

- V;

- H.

You would then obtain something like:

| Band | Polarisation | Path 1 loss | Path 2 loss | Difference |
| --- | --- | --- | --- | --- |
| B3 | V | X | Y | Y-X |
| B3 | H | X | Y | Y-X |
| B20 | V | X | Y | Y-X |
| B20 | H | X | Y | Y-X |
| B40 | V | X | Y | Y-X |
| B40 | H | X | Y | Y-X |

That would immediately tell you whether the new architecture itself contains a 2–6 dB path imbalance.

14. I would also do an FVD/path permutation test

There is an even simpler field/lab diagnostic.

You have two things that can be swapped:

RF path

and

FVD

Do the following:

Configuration A

Path 1 → FVD1
Path 2 → FVD2

Measure.

Configuration B

Path 1 → FVD2
Path 2 → FVD1

Measure again.

Then:

- if the difference follows Path 1/Path 2, suspect diplexer/cable/connector;

- if the difference follows FVD1/FVD2, suspect the FVD/RF input;

- if it partly follows both, you have both effects.

This is more diagnostic than simply swapping the cables between antenna/diplexer pairs.

15. There is another particularly important test

Because you have two polarisations, would measure the end-to-end insertion loss separately for every path, rather than only measuring the complete diplexer pair.

For example:

B3 V

Antenna → diplexer 1V → FVD1 Port 1

versus

Antenna → diplexer 2V → FVD2 Port 1

B3 H

Antenna → diplexer 1H → FVD1 Port 2

versus

Antenna → diplexer 2H → FVD2 Port 2

and repeat for B20/B40.

This is important because your results suggest that the problem might not be a simple "FVD1 path is 3 dB worse".

It may be:

Path 1 has a different frequency response from Path 2.

That would explain why the FVD1/FVD2 difference changes between B3, B20 and B40.

16. My interpretation of the three tables

If I were writing the engineering conclusion, I would say something along these lines:

The measurements demonstrate good agreement between FVD1 and FVD2 when using the original combiner/splitter configuration, with inter-FVD RSRP differences of approximately 0.1–0.3 dB under static conditions. In contrast, the new diplexer configuration introduces substantially larger inter-FVD differences, reaching approximately 5–6 dB in some measurements. The discrepancy also changes following the cable/path swap, indicating that the effect is associated with the RF distribution paths and/or their interaction with the FVD RF inputs rather than being solely attributable to propagation conditions.

Then:

The H/V imbalance should not itself be considered evidence of a fault. A static location can exhibit substantial differences between V- and H-polarised received power, particularly for the Band 40 A2G antenna because of its radiation pattern, polarisation characteristics and propagation geometry. However, the observed change in the Band 40 H/V relationship between the old and new configurations cannot be explained by the antenna/propagation characteristics alone, since the same antenna and test location are used.

And finally:

The sequential field measurements should therefore not be used to quantify the absolute insertion-loss improvement of the new configuration without an independent RF path-loss measurement. The large apparent gains, particularly the 13.6–19.35 dB observed for B3, are considerably larger than would normally be inferred solely from replacing a combiner/splitter network unless the actual measured losses of the two configurations confirm this.

I think that is a much stronger conclusion than saying simply "the new configuration has inconsistent RSRP."

One thing I would change in your current interpretation

I would not say:

"What needs to be the same is the combined V+H for both FVDs."

I would instead say:

"V and H do not need to be balanced at a given static location. What should be comparable is the end-to-end received RF level/performance at FVD1 and FVD2 when they are connected to equivalent RF paths and are receiving the same serving cell under the same conditions."

That is technically safer and harder for a colleague to challenge.

And your old configuration is excellent evidence for this: the two FVDs agree to within ~0.1–0.3 dB even though the B40 H/V imbalance itself is ~17 dB.

That is probably the most compelling observation in the entire dataset.

Bottom line

I would currently rank the explanations conceptually as:

Most strongly indicated by the data: unequal behaviour of the two new RF paths / diplexer paths / connectors / FVD RF inputs.

Plausible and expected: large B40 H/V imbalance due to the B40 antenna and propagation geometry.

Possible contributor in the active tests: UE transmission, RF isolation, desensitisation/interference and receiver state.

Not sufficient to explain Tests 6–8: live traffic, because those tests were performed with both FVDs idle.

Not supported as the main explanation: an obstacle between the test equipment and the eNodeB, because the old configuration produces very close FVD1/FVD2 results at the same static location and the new configuration changes the relationship.

The next most valuable measurement is therefore not another field RSRP test; it is an RF conducted/path-loss comparison of diplexer path 1 versus path 2, ideally followed by an FVD/path permutation test. That should allow you to separate antenna/propagation effects from the new hardware configuration very cleanly.