Is your recommendation to conduct a controlled test in the lab with a common source? would you use the Rohde & Schwarz CMW500 (I am not sure If we can get hold of it) or a simple RF source like a signal generator? In a lab we couldn't have the antenna radiating, instead the test set up would be wired.

Yes — a controlled, conducted lab test with a common RF source is exactly what I would recommend as the next diagnostic step. You do not need an antenna radiating in the lab. In fact, for the question you are trying to answer, a wired test is preferable, because it removes propagation, antenna pattern, polarisation and multipath from the experiment.

And I would start with a simple RF signal generator rather than making the CMW500 a prerequisite.

What the test is trying to establish

You are not initially trying to reproduce the complete LTE field environment.

You are trying to answer a much simpler question:

If exactly the same RF signal is presented to the two new diplexer paths, do the two paths deliver the same signal level to FVD1 and FVD2?

For that purpose, a signal generator is sufficient.

A CMW500 becomes useful if you subsequently want to investigate actual LTE/UE behaviour, rather than simply measuring the RF distribution paths.

1. Simple lab test I would do first

Something like this:

┌── Diplexer path 1 ── FVD1

RF Signal Generator ────┤

└── Diplexer path 2 ── FVD2

But you would need an appropriate splitter/coupler at the source so that the same source signal is supplied simultaneously to both paths.

For example:

┌── Attenuator ── Diplexer 1 ── FVD1

Signal Generator ── Splitter

└── Attenuator ── Diplexer 2 ── FVD2

The two branches should be made electrically equivalent as far as possible.

Then repeat at the relevant frequencies:

- Band 3

- Band 20

- Band 40

and for:

- V path

- H path

You don't actually need to transmit through the antenna.

2. But there is an important distinction: what do you measure?

There are actually two levels of testing I would consider.

Test A — RF path measurement

This is the one I would do first.

Instead of connecting the FVDs, put a power measurement instrument at the output of each path:

┌── Diplexer 1 ── Power measurement

Signal Generator ── Splitter

└── Diplexer 2 ── Power measurement

You could use a spectrum analyser with a suitable power measurement function, or an RF power meter.

Then, if you put exactly the same input into both paths:

Output Path 1 = -X dBm
Output Path 2 = -Y dBm

the difference:

Y − X dB

is directly telling you the relative loss imbalance of the two paths.

This is much cleaner than using RSRP to infer it.

3. Then Test B — use the FVDs

Once you know that the RF paths themselves are reasonably matched, you can connect:

┌── Diplexer 1 ── FVD1

RF source ── Splitter ─┤

└── Diplexer 2 ── FVD2

and see whether XCAL reports different RSRP.

This separates two possible problems:

Case 1

You measure:

Path 1 = -70.0 dBm
Path 2 = -70.2 dBm

but:

FVD1 RSRP = -80 dBm
FVD2 RSRP = -85 dBm

Then the diplexer/cable paths are essentially equal and the problem is further downstream — potentially the FVDs, their RF inputs, configuration, or the way RSRP is being reported.

Case 2

You measure:

Path 1 = -70 dBm
Path 2 = -75 dBm

and the FVDs show approximately a 5 dB difference.

Then you have a very strong indication that the new RF distribution paths themselves are responsible.

That is why I would make the direct RF power measurement the first experiment.

4. Do you need a CMW500?

No, not for the first experiment.

A CMW500 is much more sophisticated than you need if the immediate objective is simply:

"Are diplexer path 1 and diplexer path 2 electrically equivalent?"

The CMW500 is capable of LTE signalling/network emulation as well as RF generation and analysis, so it is very useful for proper UE receiver testing. R&S describes it as supporting LTE network emulation and RF generation/analysis functions.

But that sophistication isn't necessary to establish insertion-loss/path imbalance.

5. A signal generator is actually attractive for this particular experiment

For example, an R&S SMB-series generator would be perfectly suitable if you have access to one with the required frequency range. The SMB100A, for example, allows the RF output level to be set directly and has suitable RF-level control.

You could set something like:

1800 MHz → -50 dBm

and measure both paths.

Then:

800 MHz → -50 dBm

and measure both paths.

Then:

2350 MHz → -50 dBm

and measure both paths.

The exact frequencies should correspond to the centre frequencies you actually use for the relevant LTE carriers.

The absolute input level isn't particularly important for the first comparison. The relative difference between Path 1 and Path 2 is what matters.

6. There is one caveat with using a CW signal

A simple signal generator normally produces a CW tone, not an LTE waveform.

That's perfectly fine if your question is:

What is the insertion loss of the RF paths?

because you're measuring RF power through:

- cable;

- diplexer;

- connectors;

- splitter;

- etc.

But a CW signal does not reproduce the behaviour of an LTE UE receiver.

So:

CW generator

Excellent for:

RF path loss / path imbalance

but not sufficient by itself for:

LTE RSRP behaviour / UE receiver behaviour

CMW500 with LTE signalling

Useful for:

actual LTE UE behaviour

including receiver operation under controlled LTE conditions. R&S specifically supports LTE signalling and receiver testing with the CMW500.

7. I would therefore do this in two stages

Stage 1 — very simple and relatively easy

Common RF source → splitter → two diplexer paths → RF power measurement

Do this for:

- B3 V

- B3 H

- B20 V

- B20 H

- B40 V

- B40 H

and compare:

Path 1 loss vs Path 2 loss

This should be relatively quick.

You could actually test the old combiner/splitter configuration at the same time.

For example:

┌── OLD path → FVD1/input measurement

Common source ─────────┤

└── OLD path → FVD2/input measurement

and then repeat for the new configuration.

You would then have a table like:

| Band | Pol | Old FVD1 path | Old FVD2 path | New FVD1 path | New FVD2 path |
| --- | --- | --- | --- | --- | --- |
| B3 | V |  |  |  |  |
| B3 | H |  |  |  |  |
| B20 | V |  |  |  |  |
| B20 | H |  |  |  |  |
| B40 | V |  |  |  |  |
| B40 | H |  |  |  |  |

This would be extremely valuable.

8. Stage 2 — only if Stage 1 doesn't explain the problem

Then I would use a CMW500, if you can get access to one:

CMW500 LTE signal

│

▼

splitter

/     \

/       \

Diplexer 1  Diplexer 2

│           │

FVD1         FVD2

Now both FVDs see the same controlled LTE signal.

You can then examine:

- XCAL RSRP;

- RSRQ;

- SINR;

- antenna 0/1 measurements;

- serving cell;

- PCI;

- potentially throughput.

This would tell you whether the FVD measurements themselves introduce an additional discrepancy.

9. An even better experiment: swap the FVDs

I would definitely add this to the lab test.

Suppose initially:

Path 1 → FVD1

Path 2 → FVD2

You observe:

FVD1 = -80 dBm
FVD2 = -85 dBm

Now don't change the RF paths.

Swap the FVDs:

Path 1 → FVD2

Path 2 → FVD1

If you then get approximately:

FVD2 = -80 dBm
FVD1 = -85 dBm

the 5 dB difference follows the RF path.

That's very strong evidence for the path.

If instead you get:

FVD2 ≈ -85 dBm
FVD1 ≈ -80 dBm

the difference follows the FVD.

That would completely change the investigation.

10. And you can make the experiment even more powerful

Because you have two RF ports per FVD, I would do the same permutation for the ports if practical.

For example:

Path 1 → FVD1 Port 1

Path 2 → FVD2 Port 1

then:

Path 1 → FVD2 Port 1

Path 2 → FVD1 Port 1

and, separately, repeat for Port 2.

That lets you distinguish:

RF path effect

from

FVD effect

from

FVD RF-port effect.

My recommendation in your situation

I would not wait for a CMW500.

I'd ask your lab colleagues something like:

"Can we perform a conducted RF path comparison using a common signal generator and RF power measurement equipment? I would like to feed the same RF signal into the two diplexer paths and measure the output power from each path for B3, B20 and B40, for both H and V. This would allow us to establish whether there is an inherent path-loss imbalance between the two new diplexer chains independently of antenna propagation and LTE network conditions. If the paths are balanced, we could then repeat the test using the FVDs, and swap the FVDs between paths to determine whether any residual difference follows the RF path or the FVD."

That is, in my view, the most efficient next experiment.

And importantly, the fact that the lab test is wired rather than radiated is actually an advantage for this particular question. You're deliberately removing the very variables that make the field results difficult to interpret. The CMW500 is useful later if you need to reproduce actual LTE signalling; it isn't necessary to establish whether your new diplexer architecture has a 2–6 dB path imbalance.