One question relating your statement in quotes: "The large apparent gains, particularly the 13.6–19.35 dB observed for B3, are considerably larger than would normally be inferred solely from replacing a combiner/splitter network unless the actual measured losses of the two configurations confirm this." We measured the insertion loss of the old configuration which is 12dB. Is 19.35dB observed on B3 explainable?

Yes — knowing that the measured insertion loss of the old configuration is 12 dB changes the interpretation, and I would make my earlier statement more precise.

The key point

If the old configuration has 12 dB insertion loss, then the maximum possible improvement from replacing it with a completely lossless passive RF path would be 12 dB.

So:

- Old loss = 12 dB

- New loss cannot be less than 0 dB for a passive diplexer/cable network.

- Therefore, the maximum gain attributable purely to replacing the old RF distribution network is 12 dB.

Your measured B3 apparent gains are:

|  | Old RSRP | New RSRP | Apparent gain |
| --- | --- | --- | --- |
| FVD1 | -98.66 | -85.06 | +13.60 dB |
| FVD2 | -98.79 | -79.44 | +19.35 dB |

Therefore:

FVD1: +13.6 dB

This is already 1.6 dB greater than the entire 12 dB old-system insertion loss.

If everything else were identical, it cannot be explained solely by removing the old configuration's 12 dB loss.

FVD2: +19.35 dB

This is 7.35 dB greater than the entire 12 dB loss.

So it definitely cannot be explained solely by replacing a passive 12 dB-loss network with another passive network.

But there is an important nuance

This does not mean that the 19.35 dB result is "wrong".

It means that:

19.35 dB cannot be interpreted as the insertion-loss improvement of the new RF distribution system.

There must be other differences between Test 2 and Test 7 contributing to the observed RSRP difference.

For example, if the new configuration had, say, 3 dB insertion loss, then the expected improvement attributable to the RF network would be:

12 − 3 = 9 dB

But FVD2 shows:

19.35 dB

leaving approximately:

19.35 − 9 = 10.35 dB

that would have to come from other factors.

Those factors could include the field/network environment, antenna/propagation changes, UE state, measurement behaviour, etc.

There is actually an even stronger way of looking at your data

Because you know the old configuration loss is 12 dB, we can calculate an implied new insertion loss if we temporarily assume that the entire RSRP change was caused by the RF configuration.

The relationship is:

Gain = Old loss − New loss

Therefore:

New loss = Old loss − observed gain

FVD1

12 − 13.60 = −1.60 dB

That would imply a −1.6 dB insertion loss, i.e. apparent gain through a passive RF network.

That is physically impossible for your passive diplexer/cable network.

FVD2

12 − 19.35 = −7.35 dB

Even more clearly impossible.

So this is quite useful evidence.

It tells you that the field measurements cannot be used directly to calculate the insertion-loss improvement.

And this reinforces the value of the lab test

Suppose you measure the new configuration in the lab and find:

Old configuration = 12.0 dB
New configuration = 3.0 dB

Then the genuine RF-system improvement is:

12 − 3 = 9 dB

You would then expect the field measurements to show approximately a 9 dB improvement if everything else were identical.

If the field test gives:

- FVD1 = +13.6 dB

- FVD2 = +19.35 dB

then the excess is clearly coming from factors other than the passive RF path.

And importantly, the different excess for FVD1 and FVD2 would reinforce your observation that something about the two FVD paths/measurement chains is behaving differently.

One thing I would change from my previous answer

I previously said:

"the 13.6–19.35 dB observed for B3 are considerably larger than would normally be inferred solely from replacing a combiner/splitter network..."

Given your new information, I'd make that stronger and more precise:

"Since the measured insertion loss of the old configuration is 12 dB, an apparent field improvement greater than 12 dB cannot be attributed solely to replacement of the old passive RF distribution network. The +13.6 dB and particularly +19.35 dB B3 improvements therefore indicate that the difference between the sequential field measurements is influenced by factors in addition to the RF distribution loss."

I think that's a much more defensible engineering statement.

One important caveat

This assumes that the 12 dB measurement is the complete end-to-end insertion loss of the old configuration for the relevant B3 path, measured under conditions comparable to the field configuration.

If the 12 dB is, for example, only the loss of the combiner/splitter itself and excludes some cables/connectors that are different between the two measurements, then we need to account for those separately.

If you tell me exactly where the 12 dB was measured — e.g. antenna input → FVD output, or just combiner + splitter, and at what B3 frequency — I can work through the old/new RF loss budget with you. That would make the interpretation of the 13.6 and 19.35 dB results much more definitive.