I don't want L2 address to match the final destination IP's last octet. I want the L2 address last octet to match the MAC address last octet for any interface

was adding a relationship that was not ask for.

The rule wanted is simply:

For every interface, the COMSYS L2 address is represented by the last octet of that interface's MAC address. It does not need to match the final IP address.

So, for example:

Interface A

IP address:       10.0.0.1

MAC address:      02:00:00:00:00:01

L2 address:       1

Interface B

IP address:       10.0.0.2

MAC address:      02:00:00:00:00:02

L2 address:       2

The routing scenario should nevertheless be:

Final destination IP = 10.0.1.99

|

| route via

v

Next-hop IP = 192.168.0.2

|

| static neighbour mapping

v

Next-hop MAC = 02:00:00:00:00:02

|

+--> L2 address = 2

Therefore, when kaka receives the TAP frame, it sees:

Ethernet:

destination MAC = 02:00:00:00:00:02

L2 address       = 2

IP:

destination IP   = 10.0.1.99

So the three addresses are deliberately different:

L3 final destination

10.0.1.99

^

|

IP routing

|

v

Next-hop IP

192.168.0.2

|

neighbour lookup

|

v

Destination MAC

02:00:00:00:00:02

|

v

COMSYS L2 address

2

That is the correct model.

And this is the important TUN vs TAP distinction

With TUN, proto gets:

IP destination = 10.0.1.99

but no Ethernet destination MAC, so it cannot derive:

L2 address = 2

With TAP, proto gets:

IP destination = 10.0.1.99

MAC destination = 02:00:00:00:00:02

and therefore:

L2 address = MAC[5] = 2

The final destination IP and the COMSYS L2 address are independent.

So would retain the 192.168.0.x next-hop network from the earlier demo, but simplify everything else. The key test case should be:

10.0.1.99  --final destination-->

Linux routing

|

| via 192.168.0.2

v

static neighbour

|

| 02:00:00:00:00:02

v

TAP

|

v

proto

|

+--> L2 address = 2

That now matches the original requirement exactly.

*********************************************************************************

the right separation.

For this demo,  make the bash script responsible for all Linux networking configuration, including:

- creating namespaces/veths

- creating/configuring the TAP interfaces

- assigning MAC addresses

- assigning IP addresses

- disabling ARP

- configuring the route

- configuring the static neighbour entry

The C code then contains only the things that are relevant to the production proto path:

- open an already-configured TAP;

- read the complete Ethernet frame;

- extract destination MAC/L2 address;

- extract IP destination;

- forward the frame over the test transport;

- inject it into the other TAP.

There should be no system("ip ...") in the C code.

1. Topology

The test will deliberately use:

Final destination IP:  10.0.1.99

Next-hop IP:           192.168.0.2

Next-hop MAC:          02:00:00:00:00:02

COMSYS L2 address:     2

So:

Linux IP stack

|

| destination = 10.0.1.99

|

route lookup

|

| via 192.168.0.2

v

neighbour lookup

|

| 192.168.0.2

|       -> 02:00:00:00:00:02

v

TAP

|

v

proto

|

+-----------+-----------+

|                       |

dst MAC = ...02        dst IP = 10.0.1.99

|

v

COMSYS L2 = 2

Notice the important distinction:

10.0.1.99       = final L3 destination

192.168.0.2     = next-hop L3 address

...:02          = L2 address

2. setup_netns.sh (ver3)

This is where we put all of the Linux-specific setup.

ver3_setup_netns.sh

But there is a problem with this version: the TAP interfaces don't exist until the C program creates them, so we can't put the complete configuration into one linear setup script unless the C program creates the TAP first and then waits.

For the cleanest test, would be therefore make the C program do only:

TUNSETIFF -> create TAP

and have the script configure everything else after the TAP has been created.

There are two ways to do this. recommendED a very simple readiness mechanism.

3. Better setup_netns.sh

The script can start the two programs, wait for their TAP interfaces, then configure them.

Ver3_better_setup_netns.sh

There is one issue with using /tmp/proto_ready: because the namespaces have separate mount/network contexts but normally share the filesystem, it is not ideal as a synchronization mechanism.

can make this much cleaner.

4. The cleanest approach

recommendED that the C program doesn't configure anything at all other than creating the TAP.

It can simply create the TAP and then wait for a command-line signal:

proto_kernel_test A &

Then the bash script configures TAP and finally sends:

SIGUSR1

to tell the C program:

Linux networking configuration is complete; now run proto.

That makes the separation extremely obvious.

C production-relevant code

static volatile sig_atomic_t ready;

static void ready_handler(int sig)

{

(void)sig;

ready = 1;

}

After tap_alloc():

signal(SIGUSR1, ready_handler);

printf("TAP %s created. PID=%d. Waiting for network configuration...\n",

tap_name, getpid());

while (!ready)

pause();

Then the bash script does:

kill -USR1 "$PID_A"

kill -USR1 "$PID_B"

Everything before that point is test infrastructure.

5. The resulting C code is much clearer

The important point is that the C program does not know or care that Linux routing was configured with:

ip route ...

ip neigh ...

It just opens an already-created/configured TAP and receives:

Ethernet frame

The production code path is therefore basically:

int tap_fd = tap_alloc("tap0");

...

len = read(tap_fd, frame, sizeof(frame));

struct ethhdr *eth = (struct ethhdr *)frame;

l2_address = eth->h_dest[5];

...

That's exactly the separation we are looking for.

The bash test infrastructure handles:

Linux-specific setup

|

+-- TAP IP

+-- TAP MAC

+-- route

+-- next-hop

+-- neighbour table

+-- ARP disabled

while production proto handles:

TAP frame

|

+-- Ethernet destination MAC

|       |

|       +-- COMSYS L2 address

|

+-- IP packet

|

+-- final L3 destination

One important correction to the earlier demo

We should not configure 10.0.1.99 as an IP address on the same TAP that sends the packet from ns1. It belongs to the remote endpoint (tap1 in ns2).

The proper configuration is:

ns1/tap0:

10.0.0.1

192.168.0.1

MAC ...:01

route 10.0.1.0/24 via 192.168.0.2

neighbour 192.168.0.2 -> MAC ...:02

ns2/tap1:

10.0.1.99

192.168.0.2

MAC ...:02

Then the actual packet is:

Source IP       = 10.0.0.1

Final IP        = 10.0.1.99

Next-hop IP     = 192.168.0.2

Destination MAC = 02:00:00:00:00:02

COMSYS L2       = 2

That is the scenario THAT should testED.

importantly, the C code should contain zero knowledge of 192.168.0.2, 10.0.1.99, routes, neighbour entries, or Linux routing configuration. Those are exactly the things that your production higher layer will eventually configure through netlink.

The next-hop IP is still NO associated with a tap interface. The production code can read from or write into a tap interface. demo to show that when packets are routed from source to destination via a next-hop, when the packet arrives at the tap associated with the next-hop it reads the MAC address which is mapped to the COMSYS L2 as opposed to the destination final ip address.  c code to read that ethernet frame and display characteristic of TAP.  confused?

No —confusion has been in the topology WE've been proposing.

What you actually want to demonstrate is a very specific Linux behaviour:

A packet has a final destination IP, but is routed through an intermediate next-hop. At the interface representing that next-hop, kaka receives an Ethernet frame whose destination MAC identifies the next-hop/L2 endpoint, rather than containing the final destination IP as the thing from which the L2 address would have to be inferred.

That is a good test of why TAP is useful here.

The topology WE want

Something like:

Linux routing

|

Source           |                   Next-hop

10.0.0.1         |                 192.168.0.2

|                  |                           |

|                  |                           |

+---------> Linux -------------> TAP

route                        |

via                            |

192.168.0.2                 |

v

proto

But there is an important distinction:

The next-hop IP must be associated with the TAP interface on which Linux emits the packet.

That is what was missing in THE previous topology.

So the correct Linux configuration is conceptually:

TAP:

IP = 192.168.0.1

route:

final destination 10.0.1.99

via next-hop 192.168.0.2

dev tap0

neighbour:

192.168.0.2

->

02:00:00:00:00:02

Then Linux sends the packet to tap0.

The packet still contains:

IP destination = 10.0.1.99

but the Ethernet header contains:

destination MAC = 02:00:00:00:00:02

Therefore proto, reading from TAP, sees both:

L3 final destination = 10.0.1.99

L2 destination       = 02:00:00:00:00:02

|

+-- COMSYS L2 = 2

That is the exact demonstration you are after.

important point

We don't actually need a second TAP to demonstrate this.

The simplest demo can be:

Linux namespace

Application

|

| send packet to 10.0.1.99

v

Linux IP stack

|

| route:

| 10.0.1.0/24 via 192.168.0.2

v

tap0

|

| Ethernet frame

v

proto

with:

tap0:

IP = 192.168.0.1

MAC = 02:00:00:00:00:01

next-hop:

IP = 192.168.0.2

MAC = 02:00:00:00:00:02

final destination:

IP = 10.0.1.99

and the static neighbour entry:

192.168.0.2 -> 02:00:00:00:00:02

Linux therefore constructs:

+------------------------------------------------+

| Ethernet                                                |

|                                                                 |

| dst MAC = 02:00:00:00:00:02   <-- L2 = 2      |

| src MAC = 02:00:00:00:00:01            |

+------------------------------------------------+

| IP                                                            |

|                                                                 |

| dst IP = 10.0.1.99             <-- FINAL IP|

| src IP = ...                                               |

+------------------------------------------------+

Then the C program simply reads that frame from tap0 and prints:

TAP frame received

Ethernet:

destination MAC: 02:00:00:00:00:02

source MAC:      02:00:00:00:00:01

L2 address:      2

IP:

destination IP:  10.0.1.99

This is probably the simplest and clearest demo of the point.????????????????????????

There is no need for proto B, a second TAP, a veth pair, or a simulated remote machine unless you subsequently want to demonstrate the complete end-to-end forwarding.

Why this matters for THE TUN problem

THE TUN case effectively gives kaka:

IP packet

|

+-- destination = 10.0.1.99

So if THE production logic needs the L2 address for flow/subflow/queue selection, it has lost the information.

With TAP:

Ethernet frame

|

+-- destination MAC = 02:00:00:00:00:02

|                         |

|                         +-- COMSYS L2 = 2

|

+-- IP destination = 10.0.1.99

The two pieces of information are available simultaneously.

And importantly, the C code doesn't need to know anything about routing.

The bash script sets up:

tap0

|

+-- IP address

+-- MAC address

+-- route

+-- next-hop

+-- static neighbour mapping

+-- ARP disabled

The C code does:

open TAP

|

read()

|

Ethernet header

|

dst MAC -> COMSYS L2

|

IP header

|

dst IP -> final destination

That is a very clean separation between test configuration and production functionality.

One subtle terminology point

Strictly speaking, the TAP isn't "the next-hop interface" in the same way a physical Ethernet interface is. It is the Linux link-layer interface through which the packet is emitted, and the configured neighbour entry tells Linux the link-layer address of the next-hop.

For THE test, that's exactly what is wantED: the frame presented to proto on TAP contains the next-hop's MAC while the IP header still contains the ultimate destination IP.

So yes: we now understand the test wantED, and WE can simplify the demo to one TAP and put all route/neighbour/IP/MAC configuration into the bash script.

AURRERA *****************************************************************

this demo is deliberately small and keeps the boundary very clear:

- Bash = all Linux networking/routing configuration.

- C = only TAP creation plus the production-relevant operation of reading an Ethernet frame and inspecting it.

- No system("ip ...") in C.

- No netlink code in C.

- No second TAP, veth, forwarding process, or simulated remote protocol.

The test will demonstrate:

Application

|

| send to FINAL IP 10.0.1.99

v

Linux IP stack

|

| route: 10.0.1.0/24 via 192.168.0.2

|

| neighbour: 192.168.0.2 -> 02:00:00:00:00:02

v

TAP tap0

|

| Ethernet:

|   dst MAC = 02:00:00:00:00:02

|   dst IP  = 10.0.1.99

v

proto

|

+-- L2 address = MAC last octet = 2

|

+-- L3 final destination = 10.0.1.99

The important result is therefore:

Next-hop IP:       192.168.0.2

Next-hop MAC:      02:00:00:00:00:02

COMSYS L2:         2

Final destination: 10.0.1.99

1. setup_tap_demo.sh

Setup_tap_demo_with_sudo.sh

There is one practical issue with running sudo on the individual commands: if you run the whole script as root, it is cleaner to remove the sudos.

I would actually use:

sudo ./setup_tap_demo.sh

and therefore use the following cleaner version:

Setup_tap_demo_without_sudo.sh

2. proto_tap_demo.c

This is intentionally tiny.

The important thing is that there is no routing logic here.

Proto_tap_demo.c

3. But we need a source of traffic

There is one thing missing from the above: something has to cause Linux to route a packet to 10.0.1.99.

We don't want to put that into the C code because that would muddy the demonstration.

So the bash script should generate the packet after configuring the route.

The simplest way is to use ping, but because 10.0.1.99 doesn't actually exist, we only need the packet to reach TAP.

Add this after:

kill -USR1 $PID

but before wait:

ping -c 1 -W 1 $FINAL_IP || true

However, there is a subtle issue: ping generates ICMP and Linux may perform additional behaviour that isn't necessary for the demonstration.

An even cleaner test is to use a UDP packet from the command line with Python:

</> Bash

python3 - <<'PY'

import socket

s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

s.sendto(b"TEST", ("10.0.1.99", 5000))

s.close()

PY

So the end of the script becomes:

</> Bash

echo

echo "========== START TEST =========="

kill -USR1 $PID

python3 - <<'PY'

import socket

s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

s.sendto(b"TEST", ("10.0.1.99", 5000))

s.close()

PY

wait $PID

That is preferable because even the test traffic generation is outside the C code.

4. One important correction

There is a Linux routing detail we need to handle correctly.

The route:

ip route add 10.0.1.99/32 via 192.168.0.2 dev tap0

requires 192.168.0.2 to be considered reachable through tap0.

Because tap0 has:

192.168.0.1/24

Linux considers:

192.168.0.0/24

directly connected to tap0.

Therefore:

192.168.0.2

is a valid next-hop on that interface.

Then the permanent neighbour entry tells Linux:

192.168.0.2

|

v

02:00:00:00:00:02

No ARP is required.

5. What you should see

The script should show something along these lines:

========== ROUTE ==========

10.0.1.99 via 192.168.0.2 dev tap0

and:

========== NEIGHBOUR ==========

192.168.0.2 lladdr 02:00:00:00:00:02 PERMANENT

Then C should print:

============================================================

PROTO READ FROM TAP

============================================================

Frame length       : ...

Ethernet header:

Destination MAC  : 02:00:00:00:00:02

Source MAC       : ...

COMSYS L2 address : 2

IP header:

Destination IP   : 10.0.1.99

Expected result:

Final L3 destination : 10.0.1.99

L2 address            : 2

And this is the critical result:

Linux routing

FINAL IP                  NEXT HOP

10.0.1.99  ------------>  192.168.0.2

|

| neighbour lookup

v

MAC ...:02

|

v

TAP

|

v

proto

proto sees:

IP destination = 10.0.1.99

Ethernet destination MAC = ...:02

|

v

COMSYS L2 = 2

So the final IP is 10.0.1.99, while the L2 address extracted by proto is 2, because the Ethernet destination is the MAC of the next-hop, not something derived from the final destination IP.

That is the distinction you were trying to demonstrate.

Production-code boundary

This is also why I think this version is much better for your port.

Belongs in the higher layer / netlink configuration

MAC address

IP address

ARP/NOARP

route

next-hop

neighbour entry

Belongs in proto

tap_fd = tap_alloc(...);

read(tap_fd, frame, ...);

eth = (struct ethhdr *)frame;

l2_address = eth->h_dest[5];

So when you later implement the real netlink configuration, none of that routing/neighbour configuration needs to leak into tap_alloc() or the protocol data path.