Following the static analysis that revealed different RF path performance using the new diplexer configuration, we have tested the individual components and taken measurements in the lab and the anechoic chamber (antennas), including diplexers S parameters, cables, antennas (return loss,...) and seem to be within specifications and generally compliant.

So far the integration results are as expected for;

- small cables

- Long cables

- Diplexers standalone

- Antennas in the anechoic chamber (albeit not amazing return loss)

In the anechoic chamber some of the antenna couplings are stronger than the FSPL estimate. A colleague thinks that might be in the transition to far field - ie Fresnel zone?

Therefore, the issue we had observed can't be explained by different diplexer losses because those are all so small and similar, unless it is as a colleague has said and there is some interaction between diplexer, cable, and antenna port.

Subsequently, we have taken some measurements along flight paths in the air with antennas mounted on the aircraft and the test equipment installed in the aircraft. All flight paths were along the same radial (middle of the beamwidth sector) and same altitude. Flights with Ues in idle were outgoing (two devices FVD1 and FVD2), while return flights towards the eNB were with UE in active calls (FVD in continuous call and FVD2 intermitter call).

The most important flight serials were

Band 40 → 1.3.1 (old config in idle), 1.3.2 (old config in active call), 2.3.1 (new config in idle), 2.3.2 (new config in active call) at an altitude of 5000ft

Band 20 (796-801 MHz DL) → 1.2.1 (old config in idle), 1.2.2 (old config in active call), 2.2.1 (new config in idle), 2.2.2 (new config in active call) at an altitude of 1500ft

Band 3 → 1.1.1 (old config in idle), 1.1.2 (old config in active call), 2.1.1 (new config in idle), 2.1.2 (new config in active call) at an altitude of 1500ft

LTE Band 20 operates in the 800 MHz Digital Dividend spectrum. It uses Frequency Division Duplex (FDD).

The Band 3 operates in Frequency Band: 1800 MHz (DCS band). It uses Frequency Division Duplex (FDD).

The Band 40 operates in 2300 MHz (commonly known as the S-Band). It uses TDD. Duplex Mechanism: Because it is a TDD band, it is unpaired and uses a single frequency allocation of 100 MHz total width. It transmits both uplink and downlink data on the same frequency by separating them in time slots.

The results are reasonable and summarised in the tables given at the end:

- General gain in RSRP but it doesn't translate to a significant gain in SINR for B3/B20, only for B40, possibly because B3/B20 are terrestrial networks and at 15---ft the signal quality is already poor, whereas B40 is the A2G layer.

- R factor Correlation between FVD1 and FVD2 in the same serial is 0.85 for B40, 0.7 for B20 and 0.47 for B3. Note that B3 and B20, particularly B3, exhibit many handovers and this is an overall correlation for all PCIs combined. When analysing individual PCIs, sometimes there are not sufficient samples to have a reliable R correlation factor.

- The mean point-to-point between FVD1 and FVD2 is small for most cases except one large anomaly which is my main concern, specifically serial 2.3.1 with a RSRP delta between FVD1 and FVD2 of -9.75dB along the same flight path using the new diplexer configuration and in idle mode. Straight after this leg there was a return leg in (opposite direction, nose aircraft facing the eNB) and the gap between FVD1 and FVD2 is 1.8dB possibly slightly higher than expected but much lower than the previous leg. Why? This effect hasn't been observed in B20 and B3 but the signals come from different antennas and antenna cables but same diplexer paths to mix the bands. Why could Band 40 be more sensitive and why in idle mode so much more than in active mode?

- The SINR of the FVD1 that is 9.75dB lower that its partner FVD2 drops to the level of the SINR associated with the old configuration.

- RSRP level is consistent for the old combiner-splitter configuration, not just across the two FVDs but across operating modes (idle and call)

Attached 2 plots for the RSRP and SINR versus distance from serving eNodeB (same altitude), and 4 plots showing FVD1 versus FVD2 for RSRP and SINR and both modes (idle & call). The description of the eight flights including plot colour and serial number are:

label{ Charcoal } = '1.3.2, Splitter, FVD1, Continuous Call'; Return radial towards eNodeB

label{ Purple } = '1.3.1, Splitter, FVD1, Idle'; Outgoing radial away from eNodeB

label{ Terracota } = '1.3.2, Splitter, FVD2, Intermittent Call'; Return radial towards eNodeB

label{ Turquoise } = '1.3.1, Splitter, FVD2, Idle'; Outgoing radial away from eNodeB

label{ Rose } = '2.3.2, Diplexer, FVD1, Continuous Call'; Return radial towards eNodeB

label{ Blue } = '2.3.1, Diplexer, FVD1, Idle'; Outgoing radial away from eNodeB

label{ Amber } = '2.3.2, Diplexer, FVD2, Intermittent Call'; Return radial towards eNodeB

label{ Green } = '2.3.1, Diplexer, FVD2, Idle'; Outgoing radial away from eNodeB

Purple and turquoise are superimposed / overlaid

Charcoal and Terracota are superimposed / overlaid

The pair (Purple and turquoise) and (Charcoal and Terracota) are not superimposed because they are different flights but very close to each other in magnitude.

Then we have rose and Amber (return radial with FVD1 in continuous call and FVD2 in intermittent call) with correlated peaks and trough but a mean difference of 1.8 dBs

We also have the blue and green (FVD1&FVD2 in idle, outgoing) correlated in terms of peaks and troughs (albeit green at the top is very noisy) but in magnitude the shift is significant at -9.75dB

It seems that Blue and green are worrying because they flew the same leg in idle mode but they are extremely different.

The tables are as follow in CSV format:

--- Table 1: RSRP gain (new diplexer - old combiner/splitter) ---

Row Label,B40 PCI 216,B3 All,B3 PCI 291,B20 All,B20 PCI 305

FVD1 Idle,6.53,12.1,15,9.5,6.3

FVD2 Idle,16.37,12.56,12,9.6,12

FVD1 Continuous Call,14.55,10.97,10.8,9.6,7.7

FVD2 Intermittent Call,12.59,11.11,9.4,12.7,11.28

Mean,12.51,11.685,11.8,10.35,9.32

--- Table 2: SINR gain (new diplexer - old combiner/splitter) ---

Row Label,B40 PCI 216,B3 All,B3 PCI 291,B20 All,B20 PCI 305

FVD1 Idle,-1.6,0.78,3.68,-0.52,-3.5

FVD2 Idle,5.5,0.58,2.5,-0.36,0.28

FVD1 Continuous Call,8.18,0.179,-0.37,-0.94,-3.33

FVD2 Intermittent Call,5.12,-0.53,0.85,2.85,-0.09

Mean,4.3,0.25225,1.665,0.2575,-1.66

--- Table 3: R Correlation between FVD1/FVD2 ---

Row Label,B40 PCI 216,B3 All,B3 PCI 291,B20 All,B20 PCI 305

New Diplexer Idle,0.85,0.49,,0.759,

Old Splitter Idle,0.99,0.3,,0.87,

New Diplexer Call,0.76,0.56,,0.56,

Old Splitter Call,0.8,0.53,,0.63,

--- Table 4: Mean Point-to-Point Difference FVD1/FVD2 ---

Row Label,B40 PCI 216,B3 All,B3 PCI 291,B20 All,B20 PCI 305

New Diplexer Idle,-9.75,-0.11,,-0.67,

Old Splitter Idle,0.0759,0.43,,0.3,

New Diplexer Call,1.8,-0.57,,0.17,

Old Splitter Call,-0.0752,0.49,,-0.08,

Here is the full transcription of all four tables from the image, formatted as Markdown tables so you can easily copy and paste them.

1. RSRP gain (new diplexer - old combiner/splitter)

|  | B40 PCI 216 | B3 All | B3 PCI 291 | B20 All | B20 PCI 305 |
| --- | --- | --- | --- | --- | --- |
| FVD1 Idle | 6.53 | 12.1 | 15 | 9.5 | 6.3 |
| FVD2 Idle | 16.37 | 12.56 | 12 | 9.6 | 12 |
| FVD1 Continuous Call | 14.55 | 10.97 | 10.8 | 9.6 | 7.7 |
| FVD2 Intermittent Call | 12.59 | 11.11 | 9.4 | 12.7 | 11.28 |
| Mean | 12.51 | 11.685 | 11.8 | 10.35 | 9.32 |

2. SINR gain (new diplexer - old combiner/splitter)

|  | B40 PCI 216 | B3 All | B3 PCI 291 | B20 All | B20 PCI 305 |
| --- | --- | --- | --- | --- | --- |
| FVD1 Idle | -1.6 | 0.78 | 3.68 | -0.52 | -3.5 |
| FVD2 Idle | 5.5 | 0.58 | 2.5 | -0.36 | 0.28 |
| FVD1 Continuous Call | 8.18 | 0.179 | -0.37 | -0.94 | -3.33 |
| FVD2 Intermittent Call | 5.12 | -0.53 | 0.85 | 2.85 | -0.09 |
| Mean | 4.3 | 0.25225 | 1.665 | 0.2575 | -1.66 |

3. R Correlation between FVD1/FVD2

|  | B40 PCI 216 | B3 All | B3 PCI 291 | B20 All | B20 PCI 305 |
| --- | --- | --- | --- | --- | --- |
| New Diplexer Idle | 0.85 | 0.49 |  | 0.759 |  |
| Old Splitter Idle | 0.99 | 0.3 |  | 0.87 |  |
| New Diplexer Call | 0.76 | 0.56 |  | 0.56 |  |
| Old Splitter Call | 0.8 | 0.53 |  | 0.63 |  |

4. Mean Point-to-Point Difference FVD1/FVD2

|  | B40 PCI 216 | B3 All | B3 PCI 291 | B20 All | B20 PCI 305 |
| --- | --- | --- | --- | --- | --- |
| New Diplexer Idle | -9.75 | -0.11 |  | -0.67 |  |
| Old Splitter Idle | 0.0759 | 0.43 |  | 0.3 |  |
| New Diplexer Call | 1.8 | -0.57 |  | 0.17 |  |
| Old Splitter Call | -0.0752 | 0.49 |  | -0.08 |  |