is there any evidence elsewhere that indicate diplexers can introduce an intrinsic issue to this type of systems in which you combine H-pol and V-pol. COuld a diplexer introduce a distortion to one of the paths?

Yes — diplexers can absolutely introduce amplitude/phase imbalance, frequency-dependent loss, return-loss/mismatch effects, and finite inter-band isolation. But there is an important distinction:

I have not found evidence that a correctly specified diplexer has an intrinsic tendency to distort H and V differently simply because the two signals are orthogonally polarised.

A diplexer is fundamentally a frequency-selective device, not a polarisation-selective device. In your configuration, H and V are going through separate diplexer pairs, so any H/V difference would normally come from the particular RF paths/components, not from some inherent "diplexer distorts one polarisation" mechanism.

I searched specifically around this, and the literature/manufacturer information supports that distinction.

What a diplexer can do

A real diplexer has several relevant characteristics:

- Insertion loss — varies with frequency.

- Return loss / impedance matching — poor matching can cause reflections and additional loss.

- Inter-port isolation — the unwanted band isn't perfectly rejected.

- Amplitude response — can vary across the passband.

- Phase/group-delay response — can vary with frequency.

- Manufacturing/tuning differences between nominally identical units.

For example, published diplexer designs explicitly characterise insertion loss, isolation, return loss and group delay as separate parameters.

So if you have:

B3/B20 antenna → diplexer 1V → FVD1

and

B3/B20 antenna → diplexer 2V → FVD2

there is no guarantee that the two physical diplexers have exactly the same:

insertion loss vs frequency

even if they have the same nominal specification.

But your particular configuration raises an additional issue

You aren't simply using one diplexer to separate two signals.

You have two complete diplexer paths, one for each FVD:

┌─ Diplexer pair 1V ─ FVD1 Port 1

B40 V + B3/20 V ───────┤

└─ Diplexer pair 2V ─ FVD2 Port 1

and similarly for H.

So there are actually four nominally equivalent RF chains:

V path                         H path

┌─ Diplexer 1V ─ FVD1 P1      ┌─ Diplexer 1H ─ FVD1 P2

Antenna┤                              ├

└─ Diplexer 2V ─ FVD2 P1      └─ Diplexer 2H ─ FVD2 P2

If, for example, Diplexer 1V has 1 dB more B3 loss than Diplexer 2V, you get a 1 dB FVD1/FVD2 difference on B3 V.

If Diplexer 1H has 3 dB more B3 loss than Diplexer 2H, you get another difference on H.

And because the diplexer response is frequency-dependent, the imbalance could be different at:

- B3 (~1.8 GHz)

- B20 (~0.8 GHz)

- B40 (~2.3 GHz)

That is very consistent with the pattern you are seeing.

Could it actually "distort" the signal?

Yes, but I'd use more precise terminology than "distortion".

There are two different things:

1. Amplitude distortion / frequency response

This is definitely possible.

For example, imagine the two diplexer paths have:

|  | B3 | B20 | B40 |
| --- | --- | --- | --- |
| Path 1 loss | 1.0 dB | 1.5 dB | 1.2 dB |
| Path 2 loss | 2.0 dB | 4.5 dB | 2.0 dB |
| Difference | 1 dB | 3 dB | 0.8 dB |

Then the two FVDs could naturally show different RSRP differences depending on band.

This is one reason your observation:

FVD1/FVD2 difference changes between B3, B20 and B40

is important.

It is exactly the sort of thing you would expect if the two paths had different frequency responses.

2. Phase/group-delay distortion

A diplexer can also have a frequency-dependent phase/group-delay response. Published diplexer work explicitly measures group delay as part of the device characterisation.

However, I would not currently use this to explain your 5–6 dB RSRP differences.

A phase difference between two independent FVD receiver paths does not, by itself, cause a 5 dB RSRP difference because you're not combining those two paths externally.

Phase/group delay becomes much more important if signals are being recombined coherently or if the receiver's internal processing is sensitive to the resulting waveform.

For your present investigation, I'd concentrate first on:

insertion loss + return loss + isolation + frequency response

rather than phase distortion.

There is one particularly relevant point from the literature

I found a paper on diplexer design that is quite illustrative.

The authors show that when two diplexer branches are intended to provide cancellation/isolation, both amplitude and phase have to be controlled. They specifically investigate the effect of unequal attenuation between two nominally similar paths.

That's not exactly your application, so I wouldn't use it as direct evidence that your diplexers are causing the problem.

But it does reinforce the general RF principle:

Nominally identical RF paths are not necessarily electrically identical; small amplitude and phase differences matter in RF networks.

What I think is more likely in your case

I would distinguish two hypotheses:

Hypothesis A — the diplexer itself has a systematic problem

For example:

Diplexer pair 1 has substantially different insertion loss from diplexer pair 2.

This should be detectable immediately with a VNA/power measurement.

Hypothesis B — the complete new RF path is different

This could include:

- diplexer;

- cable;

- connector;

- adapter;

- termination;

- antenna feed;

- diplexer port;

- impedance mismatch.

This is actually the hypothesis I would currently favour over saying "the diplexer has an intrinsic problem."

Your cable-swap result points towards the complete RF path, but it doesn't yet isolate the diplexer itself.

And there is a very important measurement I would add

If your lab has a vector network analyser (VNA), I would actually prefer it to a simple signal generator for characterising the diplexer.

You could measure S-parameters of each diplexer/path:

For each diplexer:

- S21 / insertion loss

- S11 / return loss

- isolation between the relevant ports

- response across the whole B3/B20/B40 frequency ranges

Then compare:

Diplexer pair 1 vs Diplexer pair 2

and, importantly:

V vs H

That would give you something much more definitive than trying to infer the RF behaviour from XCAL.

I would also test the diplexers individually

This is the experiment I'd now suggest:

┌── Diplexer 1 ── measurement

Signal generator ───┤

└── Diplexer 2 ── measurement

Measure the two units independently at:

B3 → B20 → B40

Then repeat for the actual ports used in the system.

If you see something like:

Diplexer 1 B3 = 1.1 dB loss
Diplexer 2 B3 = 1.2 dB loss

but:

Diplexer 1 B20 = 1.3 dB
Diplexer 2 B20 = 4.8 dB

then you've potentially found your explanation.

Conversely, if all four diplexers are within, say, a few tenths of a dB across the bands, then we should stop blaming the diplexer and move further down the chain.

One thing I would not conclude from your current data

I wouldn't currently write:

"Diplexers introduce distortion between H and V."

That's too strong and technically not quite the right mechanism.

I'd write:

"The diplexer-based architecture introduces separate RF paths for the two FVDs. Any difference in insertion loss, frequency response, return loss or isolation between the nominally equivalent diplexer paths could result in different RSRP measurements at the two FVDs. This needs to be verified by a conducted RF measurement."

That's a much more defensible statement.

And it fits your field results very well:

old architecture → common RF path → FVD1 ≈ FVD2

new architecture → separate RF paths → FVD1 ≠ FVD2

cable swap → imbalance changes

different imbalance by frequency → possible frequency-dependent path response

The lab/VNA measurement is therefore a very good next step.