I have also received this message within square brackets from another colleague that relates to the previous messages. Does it make sense? Is it aligned with the earlier discussion? [a brief summary of the issue I mentioned on the call. I may have made a mistake here as my understanding of the protocol driver is limited in comparison, but during testing I noticed that with the new proto_output function for NG (tested with the legacy data captures) that packets were being placed in incorrect flow queues when compared with the LynxOS proto_output function. After some digging I found that because the sensor post the data was captured on was doing through-cluster routing, the destination layer 3 address would be different from the Layer 2 address that was passed into the proto_output call (via the dst structure) and as the legacy system places these packets into queues and flows based on their layer 2 addresses, whereas the new on their layer 3 addresses, they were being placed in the wrong queues - queues that had not been enabled by the PME - and therefore the packets were being dumped. I think Steves current thought is that if we used a TAP instead of TUN we can extract the layer 2 addresses and use these similar to the legacy system, but we did not want to make any changes without discussing with you first.]

Yes. This second message makes sense, and it is very closely aligned with the earlier discussion. In fact, it provides a concrete example of exactly why the distinction between the Layer-3 destination IP address and the Layer-2 next-hop address matters.

There is one important qualification, though: I would not interpret the colleague's conclusion as proving that TUN cannot work. It proves that the current TUN implementation is using the wrong information for flow/queue selection.

1. What your colleague is describing

The critical sentence is:

"the destination layer 3 address would be different from the Layer 2 address that was passed into the proto_output call (via the dst structure)"

That is entirely consistent with the legacy proto_output() you showed me.

Your legacy code does:

l2_addr = dst->sa_data[5];

and then uses l2_addr when finding/creating the appropriate subflow and queue.

So the legacy architecture is effectively:

IP packet

|

| L3 destination

v

Linux/LynxOS stack

|

| routing decision

|

+-----> dst

|

| L2 next hop

v

proto_output()

|

v

flow/subflow selection

|

v

TX queue

The important point is:

The queue selection is based on the L2 next hop, not necessarily the final IP destination.

That is exactly what your colleague has discovered.

2. The through-cluster example explains why

Suppose you have:

HLA

10.8.5.1

|

|

COMPRO

10.8.5.4

L2 = 4

|

|

TCR SENSOR

10.8.5.5

L2 = 5

|

+---------------- SENSOR 10.8.5.15

|

+---------------- SENSOR 10.8.5.19

Suppose the HLA sends:

IP source      = 10.8.5.1

IP destination = 10.8.5.19

The final destination is:

10.8.5.19

but the next COMSYS hop is:

L2 = 5

because SENSOR 19 is reached through SENSOR 5.

Therefore the legacy driver effectively sees:

IP destination = 10.8.5.19

L2 next hop = 5

and selects the queue/subflow associated with:

L2 = 5

It does not select a queue for L2 = 19.

This is precisely what your colleague means by:

"through-cluster routing"

3. This also explains the packet capture you received earlier

The earlier example was:

TX: L2 Frame: src_addr:5

PSG: dest:4

IP:src:10.8.5.5 dest:10.8.5.1

IP:src:10.8.5.15 dest:10.8.5.1

IP:src:10.8.5.5 dest:10.8.5.1

IP:src:10.8.5.5 dest:10.8.5.1

The interesting packet is:

IP source = 10.8.5.15

IP dest   = 10.8.5.1

It originates from SENSOR 15.

But the frame has:

L2 source = 5

because the transmitting node is the TCR SENSOR with L2 address 5.

Similarly, in the downlink:

RX: L2 Frame: src_addr:4

PSG: dest:5

IP:src:10.8.5.1 dest:10.8.5.19

IP:src:10.8.5.1 dest:10.8.5.15

IP:src:10.8.5.1 dest:10.8.5.5

All three packets have different IP destinations, but they're grouped into a PSG destined for:

L2 = 5

because they all need to reach the same immediate COMSYS neighbour.

That is very strong evidence for your colleague's observation.

4. And this explains the failure you saw with the new proto_output

Your colleague says:

"the new [implementation] [uses] their layer 3 addresses"

If that is literally what the new implementation is doing, then you have identified a genuine bug.

For example, suppose the new code effectively does:

l2_addr = ip->daddr & 0xff;

For:

IP destination = 10.8.5.19

that produces:

l2_addr = 19

But the correct COMSYS next hop might be:

l2_addr = 5

So the new implementation could select:

queue/L2 19

when the legacy implementation selects:

queue/L2 5

If queue 19 hasn't been enabled/configured by PME, the packet gets discarded.

That is an entirely plausible explanation for the observed discrepancy.

5. This is also exactly why your original TUN implementation needs attention

This changes the emphasis slightly from my previous answer.

Previously we established:

TUN can work if you have a mechanism to obtain the COMSYS (HCS) L2 next hop.

Your colleague's observation now demonstrates that you actually need that mechanism.

With TUN, the driver receives:

+------------------+

| IP header        |

|                  |

| dst = 10.8.5.19  |

+------------------+

It does not receive:

L2 next hop = 5

Therefore, if your TUN-based proto_output() currently uses the IP destination to select the flow, it cannot reproduce the legacy behaviour for through-cluster routing.

You need something like:

IP destination

|

v

COMSYS routing table

|

v

L2 next hop

|

v

flow/subflow selection

6. Does TAP solve this?

Potentially, but not automatically.

This is where I would slightly challenge the wording of the colleague's message:

"if we used a TAP instead of TUN we can extract the layer 2 addresses and use these similar to the legacy system"

The first part is true:

TAP gives you an Ethernet frame, so you have L2 information available.

But the second part needs more thought.

The TAP Ethernet destination MAC is not inherently the same thing as your COMSYS L2 address.

????????????????????????????????????????????????????????????????????????????????

Your COMSYS has:

L2 = 5

L2 = 19

L2 = 21

L2 = 47

whereas Linux Ethernet has MAC addresses such as:

02:xx:xx:xx:xx:05

or whatever addressing scheme you establish.

You would therefore need to ensure that the Ethernet L2 topology corresponds correctly to the COMSYS L2 topology.

In other words:

TAP Ethernet L2

|

| mapping

v

COMSYS L2

may still be required.

7. There is an even more important point

The Linux IP stack already knows something about the route.

The legacy proto_output() doesn't magically know that:

10.8.5.19 → L2 5

It gets this information through:

dst

and ultimately:

dst->sa_data[5]

So the real question isn't:

"TUN or TAP?"

It is:

"Where does the new Linux implementation obtain the equivalent of the legacy dst->sa_data[5]?"

That is the question I would put at the centre of the design discussion.

8. I would separate two routing tables

I think this is particularly important for your implementation.

You effectively have two different concepts:

IP routing

IP destination        next IP route

10.8.5.19             via ...

10.8.5.15             via ...

COMSYS L2 routing

IP destination/prefix       COMSYS L2 next hop

------------------------------------------------

10.8.5.5                    5

10.8.5.15                   5

10.8.5.19                   5

10.8.5.21                  21

224.3.1.4                   47

The second table is what the legacy implementation was effectively providing through the kernel routing infrastructure.

And that information is essential to your QoS flow selection.

9. This also explains why the PME configuration matters

Your colleague says:

"queues that had not been enabled by the PME"

This is another important clue.

Your flow architecture isn't simply:

IP destination → queue

It is more like:

protocol

+

port

+

L2 destination

+

CLP priority

|

v

flow

|

v

subflow

|

v

queue

Your legacy code demonstrates this:

subflow = flow->subflow[pri];

and then searches for:

subflow->l2_addr

subflow->src_l2_addr

So L2 address is explicitly part of the queue/subflow identity.

That makes your colleague's observation particularly significant.

10. So are the two messages aligned?

Yes, very strongly.

I'd summarise the combined conclusions as:

Message 1

The Linux implementation needs to handle COMSYS L2 routing. TUN doesn't expose L2 information, so TAP may be easier.

Message 2

We have now observed a real failure caused by the new implementation using the L3 destination where the legacy implementation used the L2 next hop.

These are describing the same underlying problem from two different angles.

The second message provides practical evidence for the first.

11. But I would NOT conclude "therefore we must use TAP"

I think that conclusion is premature.

There are three possible solutions:

Option A — TUN + explicit COMSYS L2 routing

TUN

|

IP packet

|

v

COMSYS route lookup

|

v

L2 next hop

|

v

QoS flow selection

This is my preferred approach if you want to retain the current TUN architecture.

Option B — TAP + L2 mapping

TAP

|

Ethernet frame

|

v

Ethernet L2 info

|

v

COMSYS L2 mapping

|

v

QoS flow selection

This can make L2 handling more natural but adds Ethernet semantics that your COMSYS radio/landline network doesn't actually use.

Option C — TUN + make the Linux routing configuration carry the COMSYS next-hop information

This is potentially the most elegant hybrid solution.

For example, you could have a Linux route configuration representing:

10.8.5.19/32 → COMSYS next hop 5

and have your driver obtain that information through a well-defined interface.

Then:

Linux IP destination

|

v

routing information

|

+---- IP forwarding decision

|

+---- COMSYS L2 next hop

|

v

proto_output()

That gives you TUN while preserving the semantics of the legacy dst information.

12. And this is important for hybrid interoperability

The fact that you use TUN internally does not change the COMSYS packet sent to the legacy SENSOR.

For example, the new Linux COMPRO could do:

HLA

|

| IP packet

v

TUN

|

v

proto_output()

|

| determine:

|   IP destination = 10.8.5.19

|   L2 next hop    = 5

|

v

COMSYS frame

src L2 = 4

dst L2 = 5

IP src = 10.8.5.1

IP dst = 10.8.5.19

The legacy SENSOR receives:

COMSYS frame

dst L2 = 5

IP dst = 10.8.5.19

It doesn't care that the COMPRO happened to use TUN internally.

The interoperability boundary is the COMSYS frame, not the TUN/TAP interface.

My conclusion

I think your colleague has identified a real bug/design gap, and the observation is consistent with both the legacy proto_output() and the packet captures you've shown me.

The key correction is:

The new proto_output() must not use the IP destination address as a substitute for the COMSYS L2 destination.

For through-cluster routing:

IP destination ≠ COMSYS L2 next hop

and the latter determines the flow/subflow/queue.

However:

This does not mean TUN is invalid.

It means your TUN implementation needs an explicit way of obtaining the COMSYS L2 next-hop address, equivalent to the information that legacy proto_output() obtained from dst.

If we can establish exactly how dst->sa_data[5] was populated by the LynxOS routing configuration, that is the next thing I would investigate. That will tell us whether the cleanest Linux/TUN implementation is to extend your existing routing_table_netlink_impl.c, maintain a parallel COMSYS L2 route table, or encode the information in Linux's routing configuration.

And I think this is now the most important question to resolve before changing the implementation from TUN to TAP.

I can also create an image showing the through-cluster example and exactly where the L3 destination and COMSYS L2 next-hop diverge.